Inventory

Strategic

Plan

Annual

Plan

Review and

Plan Modification

Records

NDEQ Permit Application

Review as Necessary

Review Annually

NDEQ Permit Implementation

University of Nebraska Cooperative Extension EC02-720

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Manure

Application

Workbook

Richard Koelsch, Extension Livestock Environmental Engineer

Charles Shapiro, Extension Soil Fertility Specialist

Richard DeLoughery, Extension Water Quality Educator

Developed by University of Nebraska

Cooperative Extension

Nebraska’s Comprehensive Nutrient Management Plan

(CNMP)

Manure Application Workbook

January 2004

To assist Nebraska livestock producers in developing and using their Comprehensive Nutrient Management Plan

Project Partners

University of Nebraska Cooperative Extension

Upper Elkhorn Natural Resources District

Lower Elkhorn Natural Resources District

Lower Platte North Natural Resources District

Cooperating Agencies and Organizations

Nebraska Cattlemen

Nebraska Pork Producers

Nebraska Department of Environmental Quality

USDA Natural Resources Conservation Service

Financial Partner

This material is based upon work supported by the Cooperative State Research, Education, and Extension Service, U.S. Department of Agriculture, under Agreement No. 00-51130-97816. Any opinions, findings, conclusions, or recommendations expressed in this publication are those of the author(s) and do not necessarily reflect the view of the U.S. Department of Agriculture.

Special Thanks

The authors would like to express their appreciation to the following individuals for their contributions to and review of this document:

Charles Wortmann, NU Department of Agronomy and Horticulture

Mike Kucera, Natural Resources Conservation Service

Gary Buttermore, Nebraska Department of Environmental Quality

David Tarkalson, NU Department of Agronomy and Horticulture

Bahman Eghball, USDA Agricultural Research Service, Lincoln

Ray Ward, Ward Laboratories, Inc., Kearney

Nebraska's CNMP Manure Application Workbook

If you have questions about the workbook forms or tables, feel free to contact:

Richard Koelsch

Extension Livestock Environmental Engineer

University of Nebraska-Lincoln

213 LW Chase Hall

P.O. Box 830726

Lincoln, Nebraska 68583-0726

(402) 472-4051

RKOELSCH1@UNL.EDU

Charles Shapiro

Soil Scientist — Crop Nutrition

NU Haskell Agricultural Laboratory

57905  866 Road

Concord, NE  68728

(402) 584-2803

CSHAPIRO1@UNL.EDU

Charles Wortmann

Extension Nutrient Management Specialist

154 Keim Hall

P.O. Box 830915

Lincoln, NE 68583-0915

(402) 472-2909

CWORTMANN2@unl.edu

For questions concerning Title 130 of the NDEQ regulations, contact:

Nebraska Department of Environmental Quality

Agriculture Section

P.O. Box 98922

Lincoln, NE  68509-8922

(402) 471-4239

www.deq.state.ne.us

For resources available from the Natural Resource Conservation Service visit its Web site at:

http://www.ne.nrcs.usda.gov/technical/CNMP/NE_CNMP_General.html

Table of Contents

1

### Page Number

##### Introduction

Summary of Plan Recommendations	  5

Review and Plan Modification

Post crop season summary

of yields, manure application

rate, etc.71

Annual Review

Review as Necessary

Annual Plan

Crop Available Manure Nitrogen48

Annual Field Plan for Nitrogen50

Annual Field Plan for Phosphorus52

Action Plan54

Inventory and Resources

Livestock and Poultry8

Land Application Sites12

Land Application Equipment16

Strategic Plan

Manure Nutrient Production20

Land Requirements28 1

Manure Transfer to Off-Farm Users34

Land Application Site Agreements36 2

Emergency Response Plan38

Nutrient Management Activities Plan40

Records

Manure Application Field Records59

Nutrient Status Indicator63

Continuing Education Summary64

Manure Transfer to Off-Farm Users 65

Reports of High Soil Phosphorus Levels66

Livestock Waste Discharge Notification67

NDEQ Permit Implementation

Review as Necessary

Review Annually

Reference Tables	75

NDEQ Guidance Document:

Comprehensive Nutrient

Management Plan	89

1Or Nutrient Inventory Spreadsheet available from http://manure.unl.edu/koelsch-nbalance.html

2And/or Plan for Manure Transfer to off-Farm Users

2

Introduction

What is a CNMP?

3

- What is a CNMP?  A Comprehensive Nutrient Management Plan (CNMP) is the environmental operating or management plan for a livestock or poultry facility.  It is intended to encourage efficient management of nutrients in all aspects of a livestock system, environmentally and agronomically beneficial utilization of manures, and integration of nutrient management with other key environmental issues such as odor control or soil conservation planning.

- Nutrients in manure, when managed incorrectly, represent the single largest threat to water quality from livestock production.  However, if managed correctly, manure is an environmental and agronomic asset.  Soils receiving agronomic rates of manure require less commercial fertilizer (conserving energy and limited phosphorus reserves), are higher in organic matter (contributing to greater soil productivity and water storage capacity), and may experience less runoff, erosion, and nitrogen leaching. Thus, choices made relative to the management of nutrients within a livestock operation are absolutely critical.

- What is the purpose of this workbook?  This CNMP workbook is to help a producer in the:

- preparation of a permit application to the Nebraska Department of Environmental Quality (NDEQ) for a CNMP for land application of manure;

- implementation of a nutrient management plan that maintains profitable crop production while minimizing the risk of water contamination; and the

- establishment of a record keeping system and an annual summary of recommended manure application rates that complies with NDEQ and US Environmental Protection Agency regulations.

- This workbook focuses on manure application.  Separate CNMP workbooks focus on 1) odor management planning; 2) manure and open lot runoff storage planning, operation, and maintenance; and 3) additional stewardship planning tools for whole farm nutrient management.

- How is the CNMP organized?  The Nebraska CNMP is organized according to a five-step process:

- Inventory.  Inventory records define current livestock or poultry numbers, land application sites, and available land application equipment required for an NDEQ permit.

- Strategic Plan.  The Strategic Plan documents long-term plans targeting issues specific to NDEQ permit requirements.

- Annual Plan.  The Annual Plan estimates an agronomic application rate of manure specific to individual cropping programs and guides a producer’s compliance with NDEQ requirements for manure application to not exceed agronomic rates for nitrogen.

- Documentation and Records.  Sample records can be used to verify a nutrient management plan’s successful implementation and meet all NDEQ record keeping requirements.

- Review and Plan Modification.  This final section summarizes an individual field’s nutrient balances at the end of the season and identifies potential modifications necessary for the following year’s Annual Plan.

Good

Steward

Regulation vs. Good Stewardship?  This document follows procedures accepted by NDEQ as part of its permit application and record keeping requirements.  Those worksheet or record keeping forms marked with the Nebraska Department of Environmental Quality symbol (Figure 1) contain information that is mandatory for an NDEQ permit application or follow-up NDEQ inspections.

At a minimum these planning tools (or comparable tools supplied by the producer or his/her advisor) are critical for insuring compliance with Nebraska regulations. Those worksheets or record keeping forms marked with the “Good Stewardship” symbol (Figure 2) suggest recommended planning procedures for your farm’s economic and environmental benefit.

Figure 1. The Nebraska Department of Environmental Quality Symbol.

Figure 2. The Good Steward Symbol.

4

# Summary of Management Plans and Records Required by the

# Nebraska Department of Environmental Quality

| Forms and Planning Procedures | Page # | Form # | Permit Application |  | Record Keeping |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Required | Com-pleted | Required | Currently Used |
| Introduction |  |  |  |  | — | — |
| General Information (see Storage Workbook) |  |  | X |  | — | — |
| Summary of Strategic Plan Recommendations | 5 | 1 |  |  | — | — |
| Inventory |  |  |  |  | — | — |
| Livestock and Poultry Inventory for Meat Production Livestock Inventory for Milk, Egg and Wool Production | 8 10 | 2 3 | X or X |  | — | — |
| Inventory of Land Application Sites | 12 | 4 | X |  | — | — |
| Land Application Equipment Inventory | 16 | 5 | X |  | — | — |
| Strategic Plans |  |  |  |  | — | — |
| Manure Nutrient Production Book Values Method Nutrient Balance Method Nutrients Available After Storage Loss Nutrients Available After Land Application Loss | 20 22 24 26 | 6 7 8 9 | X X X X |  | — | — |
| Land Requirements for: Manure Nitrogen Utilization Manure Phosphorus Utilization Sludge P Utilization from Anaerobic Lagoon | 28 30 32 | 10 11 12 | X — X |  | — | — |
| Manure Transfer to Off-Farm Users | 34 | 13 | X |  | — | — |
| Land Application Site Agreements | 36 | 14 | X |  | — | — |
| Emergency Response Plan | 38 | 15 | X |  | — | — |
| Nutrient Management Activities Plan | 40 | 16 | X |  | — | — |
| Annual Plan |  |  |  |  | — | — |
| Crop Available Manure Nitrogen | 48 | 17 | — | — | X |  |
| Annual Field Plan for Nitrogen | 50 | 18 | — | — | X |  |
| Annual Field Plan for Phosphorus | 52 | 19 | — | — |  |  |
| Action Plan | 54 | 20 | — | — | X |  |
| Documentation and Records | 57 |  | — | — |  |  |
| Manure Analysis Reports |  |  | — | — | X |  |
| Soil Test Reports |  |  | — | — | X |  |
| Manure Application Field Records Solid Manure Slurry or Sludge Towed Hose or Irrigation Irrigation | 59 60 61 62 | 21 22 23 24 | — | — | X or X or X or X |  |
| Crop, Soil, Water Nutrient Status Indicators | 63 | 25 | — | — |  |  |
| Continuing Education Summary | 64 | 26 | — | — |  |  |
| Manure Transfer to Off-Farm Users | 65 | 27 | — | — | X |  |
| Reports of High Soil Phosphorus Levels | 66 | 28 | — | — | X |  |
| Livestock Waste Discharge Notification | 67 | 29 | — | — | X |  |
| Review and Plan Modification |  |  | — | — |  |  |
| Post Cropping-Season Summaries of Manure Analysis and Application Rate | 72 | 30 | — | — |  |  |
| Post Cropping-Season Summaries of Crop Yield | 73 | 31 | — | — |  |  |

Good

Steward

5

Introduction

Form 1. Summary of Strategic Plan Recommendations

# These pages are to be completed after completing the strategic plan.

| # Animal Feeding Operation Name: |  |
| --- | --- |

# Manure Nutrient Inventory Summary

# A.  Summarize Nutrient Excretion from Form 6 or 7 (page 21 or 22) or Nutrient Inventory spreadsheet.

# Nitrogen	P2O5

# Manure Handling System 	(lbs./year)	(lbs./year)

| 1. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2. |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |
| Totals |  |  |  |  |  |  |

# B.  Annual Available Nutrients After All Losses from Forms 8 and 9 (pages 25 and 27) or Nutrient Inventory spreadsheet.

| Manure Handling System |  |  |  | Nitrogen (lbs./year) |  | P2O5 (lbs./year) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |
| Totals |  |  |  |  |  |  |

| # C. Land Requirements for Managing Nutrients from Forms 10, 11, and 12 or Nutrient Inventory spreadsheet. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | # For N Management (Form 10, Column c) |  |  | # For P Management (Form 11, Column c) |  |  | # For Sludge P Management (Form 12, Column c) |  |
| # Crop 1: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Crop 2: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Crop 3: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Crop 4: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Crop 5: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Crop 6: |  |  |  | # acres |  |  | # acres |  |  | # acres |
| # Totals: |  |  |  | # acres |  |  | # acres |  |  | # acres |

# Years between

# sludge removal:       yrs.

Good

Steward

6

Form 1. Summary of Strategic Plan Recommendations (continued)

# Summary of      	:  (Form      	)

# Summary of      	:  (Form      	)

# Summary of      	:  (Form      	)

NDEQ Permit Application

NDEQ Permit Implementation

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Inventory of Resources

Inventory		page

Livestock	8

Land Application Sites	12

Land Application Equipment	16

Strategic

Plan

Annual

Plan

Review and

Plan Modification

Records

7

An electronic copy of this workbook and individual worksheets are available online at http://cnmp.unl.edu

Review as Necessary

Review Annually

8

Instructions for Livestock and Poultry Inventory

for Meat Production (Form 2)

Required Information:  Information requested by the first five columns (a through e) in Form 2 is required for an NDEQ Livestock Waste Control Facility (LWCF) Permit application.

Col. a.  Describe each production animal “Species and Group”

Species	Grouping examples

Pork	Nursery, grow/finish, gilts

Cattle	Calves, feeder

Poultry	Broilers, pullets

Turkey

Col. b.  Describe housing type used for managing animals (e.g. open lot, freestall barn, slatted floor barn) and a reference location (identified on a map of facilities) for where this animal group is typically housed.

Col. c.  Enter the maximum one-time animal population for this group of animals. (Do not use the annual throughput or number of animals finished in this facility.)

Cols. d and e. List the average weight of animals entering and leaving this animal group.

Optional Information:  To accurately estimate the quantity of nutrients excreted annually, the remaining information requested is valuable but not required by NDEQ.

Cols. f and g.  Enter “Average Days on Feed” and “Turns per Year”.

Cols. h - m. Enter Daily Feed Intake, based on maximum one-time population, and Feed Composition.  Moistures for feed intake and composition can be entered on an “As Fed” or “Dry Basis”.  However, both intake and composition must be on the same moisture basis.  CP = crude protein, P = phosphorus, K = potassium.

Col. n. For pork finishing production only, enter average Fat Free Lean Index of marketed hogs.  This value is used to estimate nitrogen retention and excretion by grow/finish hogs.

Form 2.  Livestock and Poultry Inventory for Meat Production

Inventory of animals (in confinement housing or open lots) fed for meat production, replacements, or reproduction.

| Information in Columns a-e is required for the NDEQ permit application. |  |  |  |  | Information in Columns f-n is not required for permit application. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a. Species and Group | b. Describe Confinement and Location | c. Maximum One-Time Capacity (# of animals) | Average Weight (lbs.) |  | f. Average Days on Feed | g. Turns per Year | Daily Feed Intake |  | Feed Composition1 |  |  |  | n. Fat Free Lean Index2 |
|  |  |  | d. Begin | e. End |  |  | h. Feed (lbs./day)1 | i. Moisture Basis X | j. % CP | k. % P | l. % K | m. Moisture Basis X |  |
| Example: Pigs/Finish | Slatted floor barn…Barn 1 | 1,000 | 45 | 250 | 110 days | 3 | 5,350 lbs. | As Fed Dry | 17% | 0.6% | 1% | As Fed Dry |  |
| 1. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 2. |  |  |  |  |  |  | As Fed | As Fed Dry Dry Basis |  |  |  | As Fed Dry Dry Basis |  |
| 3. |  |  |  |  |  |  | As Fed | As Fed Dry Dry Basis |  |  |  | As Fed Dry Dry Basis |  |
| 4. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 5. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 6. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 7. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 8. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 9. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |
| 10. |  |  |  |  |  |  | As Fed | As Fed Dry |  |  |  | As Fed Dry |  |

- Both daily feed intake and feed composition should be measured on the same moisture basis (e.g. both on an “As Fed” basis).

9

Fat Free Lean Index is needed only for market hog.  This measure should be available for market hogs at the time of slaughter.

Inventory

10

Instructions for Livestock and Poultry Inventory

for Milk, Egg, and Wool Production (Form 3)

Required Information:  Information requested by the first five

columns (a through e) is required for an NDEQ Livestock Waste Control Facility Permit application.

Col. a. Describe each production animal species and group:

Species	Grouping examples

Dairy	Lactating, dry, replacement heifer, bull

Pork	Gestating sow, sow and litter, boar

Cattle	Cow, bull

Poultry	Layers

Turkey

Col. b. Describe housing type used for managing animals (e.g. open lot, freestall barn, slatted floor barn) and a reference location (identified on a map of facilities)  for where this animal group is typically housed.

Col. c. Enter the maximum one-time animal population for this group of animals. (Do not use the annual throughput or number of animals finished in this facility.) Average is not required.

Col d. List the days per year that the facility is occupied.

Col. e. List  the average weight of animals in this animal group.1

Optional Information:  To accurately estimate the quantity of nutrients excreted annually, the remaining information requested is valuable but not required by NDEQ.

Cols. f - g. Enter the average annual production levels for wool, milk, and eggs.

Cols. h - i. Enter daily feed intake based on the maximum one-time population.  Feed intake and composition can be entered on an “as fed” or “dry basis”; however, both intake and composition must be on the same moisture basis.

Cols. j - m.  Feed composition in percentage: CP = crude protein,       P = phosphorus, K = potassium

Form 3. Livestock and Poultry Inventory

for Milk, Egg, and Wool Production

| Information in Columns a-e is required for the NDEQ permit application. |  |  |  |  | Information in Columns f-n is not required for permit application. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a. Species/Group | b. Describe Confinement and Location | c. Maximum One-Time Capacity (# of animals) | d. Days per Year Facility is Occupied | e. Average Weight (lbs./an.) | Average Production |  | Daily Feed Intake1 |  | Feed Composition1 % |  |  |  |
|  |  |  |  |  | f. Lbs./ animal/yr | g. Product | h. Feed (lbs./day)1 | i. Moisture Basis X | j. % CP | k. % P | l. %K X | m. Mois-ture |
| Example: Dairy/lactating cows | Freestall…Barn 3 | 500 | 365 | 1425 lbs. | 27,000 | Milk | 5,350 lbs. | As Fed Dry | 16% | 0.55% | 5,350 lbs. | As Fed Dry |
| 11. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 12. |  |  |  |  |  |  |  | As Fed Dry Dry Basis |  |  |  | As Fed Dry Dry Basis |
| 13. |  |  |  |  |  |  |  | As Fed Dry Dry Basis |  |  |  | As Fed Dry Dry Basis |
| 14. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 15. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 16. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 17. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 18. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 19. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |
| 20. |  |  |  |  |  |  |  | As Fed Dry |  |  |  | As Fed Dry |

- Both daily feed intake and feed composition should be measured on the same moisture basis (both on an “as fed” basis or both on a “dry weight” basis).

2.   an. = animal

11

Inventory

Instructions for Inventory of Land Application Sites

12

for Manure Application (Form 4, Part 1)

Complete the information requested in the Inventory of Land Application Sites section for each site where manure is likely to be distributed.  Reproduce sufficient copies of Form 4, part 1 for each potential land application site.  NDEQ should be notified of future changes in land application site information.

Field ID:  Provide an identifying name or number for each field.  Also indicate useable area in acres from which a crop is harvested.  CRP fields may be available for manure applications, especially if they were released and harvested for hay under the Emergency Haying and Grazing Program.  NRCS permission is needed for manure application.

Location:  Provide a legal description for each field site.  The location of each application site should be identified on the community siting map of the Odor Management Plan Workbook (Form 1).

Ownership:  Indicate ownership of land application site, including owner's name and address. Signed Land Application Site Agreements are required for listed land application sites not owned by the operation.  These access agreements also are required when access to an application site requires animal manure to be conveyed across property not owned by the operation, including county rights-of-way.  Note: Responsibility for liability due to damages caused by a manure application applied by livestock owners to land under an agreement remains with the livestock producer.

Availability for Manure Application: Number of years available. Mention any restrictions.

Cropping Practices: List each crop in your planned rotation and the historic yields.  Documentation is required to support yields used in determining crop utilization of nitrogen in animal manure.  If no yield information is available, state or county five-year averages for crop yields must be used (Nebraska Agricultural Statistics Service, Nebraska Department of Agriculture).

Soil Type: A soil series is a name, like Sharpsburg or Valentine.  Texture is the predominant proportion of  sand, silt, and clay, for example: silty clay loam, loamy sand, etc.  The county soil survey has this information.  Slope is designated in the Soil Survey as a capital letter between B and F.  Each letter represents a range of slope in percent.  If there is no letter, then slope is assumed to be near zero.  See the Soil Legend page in the Soil Survey for the percentage.  Primary and Secondary Soils refer to most common and second most common soil types found within the land application site.

Distance to Nearest Body of Water: This is estimated as the distance between the edge of field and the nearest surface water down gradient of the field.  NDEQ may restrict any livestock manure application "within 30 feet of any streams, lakes and impounded waters..." in the field.  NDEQ also may require some conservation practices for manure applications within 100 feet of these waters. If there is any question about stream classification, visit your NRD or NRCS office to view the USGS topographic map.

From NDEQ Title 130, Ch.1, 047: “ ‘Surface waters’ shall mean all streams, lakes, ponds, impounding reservoirs, marshes, wetlands, watercourses, waterways, springs, canal systems, drainage systems, and all other bodies or accumulations of water, natural or artificial, public or private, situated wholly or partly within, or bordering upon, the State.  Impounded waters in this definition do not include areas designated by the Department as wastewater treatment or wastewater retention facilities or irrigation reuse pits.”

If there are any wetlands (as determined by NRCS) on the operation or application areas, then their locations need to be provided to NDEQ (use the map for locations).  If there is any question about the status of a wet area, request a Certified Wetlands Determination from the county NRCS.  Acres in wetlands are to be excluded from reported useable acres.

Soil Test Phosphorus:  This is your 'base-line' record of soil phosphorus levels (not required for NDEQ permit application).  Include copies of the lab soil test reports in the Documentation and Records section.  Sample areas generally should be no more than 40 acres, with a 6- to 8-inch core depth.

13

Form 4 (part 1). Inventory of Land Application Sites

for Manure Application

| Inventory Field ID: |  |  |  |  |  |  |  | Field Size: |  |  |  |  | useable acres |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ¼ |  | Section |  | Township |  | Range |  |  | E or W |  | County |  |  |

| Other Manure Sources: Is livestock manure from another facility applied to this site? | YES | NO |
| --- | --- | --- |

If yes, attach manure production information from the other facility.

| Ownership: | Own | Rent | Neighbor | Is there a site agreement? | YES | NO |
| --- | --- | --- | --- | --- | --- | --- |

If  “Rent” or “Neighbor” is checked, list name and address of landowner and attach Application Agreement:

| Name(s) |  |  |  |  | Address(es) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cropping Practices: Planned rotation: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Five year average yields: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source of yield values: Irrigated1 | FSA verified yields No Flood Sprinkler |  |  |  |  |  |  | or other acres |  |  |  |  |  |

| Environmental Considerations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Is there a USDA approved conservation plan for this site? |  |  |  |  |  |  |  |  | YES |  | NO |  |  |  |  |  |  |
| Current Conservation Practices: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil Type: | Series and Texture |  |  | Slope (avg.) |  |  |  | Soil Phosphorus Levels1 |  |  |  |  |  |  |  |  |  |
| Primary soil: |  |  |  |  |  |  | % | Management Areas |  |  |  |  |  | Soil Test |  |  |  |
| Secondary soil1: |  |  |  |  |  |  | % | Name/ID |  |  |  | Acres |  | P (ppm) | Method* |  |  |
| Distance to nearest body of water: |  |  | feet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Does field contain: |  |  | YES |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |
| Highly erodible land? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Perennial stream? |  |  |  |  |  |  |  | Sample depth inches |  |  |  |  |  |  |  |  |  |
| Intermittent stream? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concentrated flow? |  |  |  |  |  |  |  | * B =Bray 1, weak acid M = Mehlich III O = Olsen (sodium bicarbonate). Write in test name if not one of these. |  |  |  |  |  |  |  |  |  |
| Flooding/overflow potential? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Designated wetlands? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acres of wetlands: |  |  | acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1Not required for NDEQ permit application.

Form 4 (Part 2).  Land Application Sites

14

for Manure Application

Field Map of Current Conservation Practices:  For each land application site, complete a scaled drawing or attach an aerial photograph (e.g. soils map, USGS topographic map or USDA FSA aerial photo) showing field location.  The following items should be clearly marked on each field:

Legend

Bottom 40

Field boundaries and field ID.  Check the field ID on

the map for a match with the field ID listed in Form 4, Part 1.

- Identify location of all surface water features, including:

- perennial (continuous) streams;

- intermittent streams;

- drainage tiles;

- drainage ditch;

- small pond, reservoir or wetland; and

- designated wetlands.

W

3.	Identify location of all groundwater connections, including:

DW

well locations;

- drainage wells; and

- rock outcrops.

4.	Identify current conservation measures and locations, including:

- grassed waterway; and

other (e.g. areas of no or reduced tillage, terraces, grass filter strip, setback areas with no manure application, etc.).  Clearly label each conservation practice.

e.g.  one pass tillage

5.	Nebraska operating permit: A 30-foot minimum setback is required between the edge of a stream channel, pond, or lake and the manure application. Setbacks of 100 feet are sometimes required. No setback is required from wetlands or grass waterways if they are not part of a stream or lake, but manure cannot be applied to standing or flowing water.  Setback distances are  required for some intermittent streams.  Check with NDEQ.  Setback distances are not required wells, buried tile lines, wetlands, irrigation reuse pits, manure storage facilities, or grassed waterways.

EPA’s NPDES permit will require setbacks of 100 feet or a permanent vegetated buffer of 35 feet between areas of manure application and down-gradient surface waters, open tile line intake structures, sinkholes, agricultural well heads, or other conduits to surface waters.

Form 4 (Part 2):  Field maps of Manure Application Sites

15

Inventory

Create a scaled drawing on this page or replace with USDA-FSA aerial photograph for each land application site.  Attach to Form 4, Part 1 (page 13).

16

Land Application Equipment Inventory

Instructions for Form 5

Purpose

Form 5 is designed to summarize the availability of equipment for land application of manure and related descriptive information.  NDEQ requests this information as part of its cursory review of the ability of a livestock operation to handle the volume of manure (based on animal numbers) and the transport distances (based on land application site information) indicated by a permit application.

### Instructions

For each piece of equipment used for application of manure or lagoon effluent (spreader, slurry tank, irrigation system, towed hose unit), please identify the following information.  For custom application equipment, list the most likely custom application equipment that normally will be used:

Column a.  Indicate a name or description for each piece of land application equipment.

Column b.  Type of Equipment:  slurry tank…check “ST”;  solids spreader…check “SS”;  center pivot…check “CP”; other sprinkler…check ”OS”;  flood irrigation…check “FI”;  towed hose tractor unit…check  “TH”

Column c.  Does equipment include applicator that allows immediate incorporation into soil?

Column d.  What is the capacity of this equipment for hauling or pumping?

Column e.  What is a typical total daily application rate for this piece of equipment?

Column f.  Who owns this land application equipment?  If Owned by livestock operation…check  “O”;  if Leased by livestock operation…check “L”;  if equipment belongs to a Custom Applicator… check “CA”.

Column g.  Which livestock system(s) or storage system(s) is (are) served by this equipment for land applying manure?  For example, a box spreader might be used for land applying scraped manure from the feedlot pens and settle solids from the settling basin.  A center pivot might be used for land applying top water from an anaerobic lagoon.

Form 5. Land Application Equipment Inventory

| a. X Equipment Description | b. Type of Equipment | c. Includes Incorporation Attachment? | d. Capacity |  | e. Typical Total Daily Application Rate |  | f. Ownership of Land Application Equipment 1 | g. Manure Storage System Served by this Equipment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example: Slurry Tank Spreader X X X X | ST SS CP OS FI TH | YES NO | 3,000 | tons/load | 30 loads in 10 hrs. or 90,000 | tons/day | O L CA | ### Swine finish, ### Deep pits |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |
|  | ST SS CP OS FI TH | YES NO |  | tons/load |  | tons/day | O L CA |  |
|  |  |  |  | gallons/min. |  | gallons/day |  |  |
|  |  |  |  | gallons/load |  | acre-in./day |  |  |

_______________________

1. Name and address of custom applicator:

17

Inventory

18

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Strategic Plan

Inventory

Strategic Plan

Manure Production	20

Land Requirements	28 1

Manure Transfer to Off-Farm Users	34

Land Application Site Agreements	36

Emergency Response Plan	38

Nutrient Management Activities Plan	40

Annual

Plan

Review and

Plan Modification

Records

NDEQ Permit Application

Review as Necessary

Review Annually

NDEQ Permit Implementation

1Or Nutrient Inventory Spreadsheet available from http://cnmp.unl.edu/cnmpsoftware.html

An electronic copy of this workbook and individual worksheets are available online at http://cnmp.unl.edu

19

20

Manure Nutrient Production

Instructions for Book Value Method  of Estimating Nutrient Excretion (Form 6)

Purpose

Forms 6 through 12 estimate the land requirements for managing the nutrients in manure. Having access to sufficient land to utilize the nutrients in manure is fundamental to reducing environmental risk associated with manure management.  The first step is to estimate the quantity of nutrients excreted by livestock.  Form 6 provides a method for estimating nutrient excretion generally accepted by NDEQ.

Regulations

The state of Nebraska requires that a livestock operation demonstrate access to sufficient land to manage the nitrogen in manure.  Although land requirements are not directly related to phosphorus in manure, Nebraska regulations trigger an NDEQ review of land application sites when soil phosphorus levels exceed 150 ppm measured by the Bray 1 or Mehlich III soil tests (or 100 ppm by Olsen soil test).  To avoid this trigger, sufficient land is critical for managing the phosphorus in manure.

References

Manure Nutrient Inventory Spreadsheet available online at:   http://cnmp.unl.edu/cnmpsoftware.html2

Estimating Manure Nutrients from Livestock (NebGuide G97-1334 available at http://www.ianr.unl.edu/pubs/wastemgt/g1334.htm)

### Instructions

Step 1.  Select between two alternative methods for estimating total manure nutrient excretion.  The two methods are: 1) the Book Values Method (Form 6) and the Nutrient Balance Method (Form 7).  The Book Values Method uses standard values for nutrient excretion and is commonly utilized by NDEQ in the review of permit applications.  The Nutrient Balance Method (Form 7) estimates nutrient excretion based on feed intake, feed nutrient concentration, and nutrients retained by the animal for meat, milk or egg production.  This alternative estimate is generally a more accurate estimate of nutrient excretion and will give credit to dietary changes designed to reduce nutrient excretion.  The referenced Excel spreadsheet performs the same calculations as in Forms 6 and 7.

Instructions for Book Values Method (Form 6):

Step 2.  Identify the manure storage system used on this livestock operation.  A separate worksheet should be used for each manure handling system used in this livestock operation.  For example, if the manure from the sow herd feeds an anaerobic lagoon and the manure from the grow/finish hogs is stored in a below-floor pit, use two separate worksheets, one for the anaerobic lagoon and one for the below-floor pits.  The totals should be kept separate.

Step 3. Enter the required data:

Enter the following values for each group of livestock supplying manure:

Col. b:  Maximum one-time animal population supplying manure to the selected manure storage system.

Col. c:  The average body weight for each species and animal group.

Col. e:  The decimal fraction of the year the facility is occupied. NDEQ requires use of 1.0 to represent maximum capacity.

Step 4:  Complete the following calculations:

Col. d: Total animal weight   =  Columns b x c.

Col. g: Total nitrogen production   =  Columns d x e x f.

Col. i: Total phosphorus production (as phosphate) = Columns d x e x  h.

Step 5:  Total nitrogen and phosphorus production.  Add manure nutrient production for all groups of animals using the same manure handing system and record it in:

Cell j.  Total nitrogen produced, and

Cell k. Total phosphorus produced (as phosphate).

Form 6.  Manure Nutrient Production:

Book Values Method for Estimating  Manure Nutrient Excretion by Livestock

Manure storage system:

| a. Livestock or Poultry Species | b. Maximum One-time Capacity (# of animals) | c. Average Weight (lb./animal) | d. Total Animal Weight at Capacity (lbs.) (b x c) | e. Fraction Of Year Facility is Occupied1 | f. Lbs. of Manure N per lb. of Animal Weight per Year | g. Total N Production lbs. N/yr. (d x e x f) | h. Lbs. of manure P2O5 / lb. of Animal Weight per Year | i. Total P2O5 Production lbs. P2O5 /yr. (d x e x h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example : Swine...Finish | 2000 | 150 | 300,000 | 1.0 | 0.15 | 45,000 | 0.13 | 39,000 |
| Swine Nursery |  |  |  |  | 0.22 |  | 0.21 |  |
| Grow |  |  |  |  | 0.15 |  | 0.13 |  |
| Finish |  |  |  |  | 0.15 |  | 0.13 |  |
| Sows and Litter |  |  |  |  | 0.17 |  | 0.12 |  |
| Sows (Gestation) |  |  |  |  | 0.07 |  | 0.05 |  |
| Gilts |  |  |  |  | 0.088 |  | 0.066 |  |
| Boars |  |  |  |  | 0.055 |  | 0.042 |  |
| Beef (450-750 lb.) |  |  |  |  | 0.11 |  | 0.083 |  |
| Beef Feeder (high energy diet) |  |  |  |  | 0.11 |  | 0.078 |  |
| Beef Feeder (high forage diet) |  |  |  |  | 0.11 |  | 0.091 |  |
| Beef Cow |  |  |  |  | 0.12 |  | 0.10 |  |
| Dairy Cow...50 lb./d |  |  |  |  | 0.18 |  | 0.087 |  |
| Dairy Cow...70 lb./d |  |  |  |  | 0.22 |  | 0.096 |  |
| Dairy Cow...100 lb./d |  |  |  |  | 0.27 |  | 0.110 |  |
| Dry Cow |  |  |  |  | 0.11 |  | 0.074 |  |
| Heifer/Calves |  |  |  |  | 0.11 |  | 0.033 |  |
| Layer |  |  |  |  | 0.30 |  | 0.26 |  |
| Pullet |  |  |  |  | 0.23 |  | 0.20 |  |
| Broiler |  |  |  |  | 0.40 |  | 0.28 |  |
| Turkey |  |  |  |  | 0.27 |  | 0.23 |  |
| TOTALS: |  |  |  |  | j. Total N production: lbs. N/yr. |  | k. Total P2O5 production: lbs. P2O5 /yr. |  |

1.   NDEQ typically requires use of 1.0 to represent maximum capacity.

21

Source:  NRCS Agricultural Waste Management Handbook, 4/92 with exception of dairy lactating and dry cows.  Dairy estimates are from H.H. Van Horn.  1991.  Achieving environmental balance of nutrient flow through animal production systems.  The Professional Animal Scientist. 7:3:22-33.

Strategic Plan

Good

Steward

Manure Nutrient Production

22

Instructions for Nutrient Balance Estimate of

Nutrient Excretion (Form 7)

Feed Nutrient  Nutrients Retained by Animals Nutrient

Intake or Animal Products Excretion

Feed

Manuree

Purpose

Form 7 provides an alternative Nutrient Balance Method of estimating nutrient excretion based upon animal feed nutrient intake. It is generally considered more accurate but a less widely accepted approach at the time this publication was prepared.  Typically, NDEQ uses procedures found in Form 6 to estimate nutrient excretion.

References

Nutrient Inventory Spreadsheet available online at: http://cnmp.unl.edu/cnmpsoftware.html

### Instructions

Step 1.  Select between two alternative methods for estimating total manure nutrient excretion (see Form 6 for Book Values approach discussion). Nutrient Balance Method (Form 7) estimates nutrient excretion based upon feed intake, feed nutrient concentration, and nutrients retained by the animal for meat, milk or egg production.  This alternative estimate generally provides a more accurate indication of actual nutrient excretion and will give credit to dietary changes designed to reduce nutrient excretion.  The referenced Excel spreadsheet performs the same calculations as Form 7.

Instructions for Nutrient Balance Method (Form 7)

Step 2.  Identify the manure storage system used on the livestock operation.  A separate worksheet should be used for each manure handling system.  For example, if the manure from the sow herd feeds an anaerobic lagoon and the manure from the grow/finish hogs is stored in a below floor pit, two separate worksheets should be used, one for the anaerobic lagoon and one for the below floor pits.  The totals should be kept separate.

Step 3. Enter the required data:

Enter the following values for each species of livestock supplying manure to this system:

(Values for Columns b, c, e, h, i, and n may be found in Inventory Forms 2 and 3)

| - Col. a: Animal group to be evaluated | - Col. i: (meat production only) Average daily gain per |
| --- | --- |
| - Col. b: Daily feed intake (lbs.); note 'as fed' or 'dry' | animal [Form 2: Cols. (e - d) ÷ f ] (lbs. gain/day) |
| - Col. c: Average feed crude protein (CP) content (%) | - Col. n: (milk and egg production only) Daily |
| - Col. e: Average feed phosphorus (P) content (%) | production [Form 3] (lbs. milk or eggs/day) |
| - Col. h: (meat production only) Maximum one-time capacity: number of animals | - Col. s: Days fed per year [Forms 2 or 3] |

Step 4.  Complete the following calculations:

- Col. d:  Feed nitrogen concentration   =  Col. c   ÷   6.25  (converts quantity of protein to nitrogen)

- Col. f:   Daily nitrogen in feed  =  Col. b   x   Col. d   ÷  100

- Col. g:  Daily phosphorus in feed  =  Col. b   x   Col. e   ÷  100

- Col. l:   Daily nitrogen retained by animals  =  Col. h   x   Col. i   x   Col. j

- Col. m: Daily phosphorus retained by animals  = Col. h   x   Col. i   x   Col. k

- Col. q:  Daily nitrogen retained by animal products  =  Col. n   x   Col. o

- Col. r:  Daily phosphorus retained by animal products  =  Col. n   x   Col. p

- Col. t:  Annual nitrogen excretion =  [Col. s  x  (Col. f  - Col. l)]  OR  [Col. s  x  (Col. f  - Col. q)]

- Col. u:  Annual phosphorus excretion  = [Col. s  x  (Col. g  - Col. m)]  OR  [Col. s  x  (Col. g  - Col. r)]

- Col. v:  Annual P2O5 (phosphate form) excretion   = Col. u  x  2.3

Step 5.  Add together manure nitrogen and phosphorus production for Farm Total.  Sum the values in Column t and put it in Cell w (total nitrogen production), and sum the values in Column v and put it in Cell x (total phosphorus production). This is the excretion for all groups of animals supplying the same manure handing system.

23

Good

Steward

Form 7. Manure Nutrient Production:

Nutrient Balance Estimate of Manure Nutrient Excretion

# I. Feed Nutrient Intake	Manure storage system:

| a. Animal Group | b. Group Daily Feed Intake (lbs./day) | Feed Nutrient Concentration (%) |  |  | Total Nutrient in Feed (lbs./day) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | c. Crude Protein | d. N1 (c ÷ 6.25) | e. P | f. Daily N fed (b x d ÷ 100) | g. Daily P fed (b x e ÷ 100) |
| Example: 1,000 beef finisher | 27,000 lbs. DM/day | 13.5% | 13.5 ÷ 6.25 = 2.16% | 0.35% | 27,000 X 2.16 ÷ 100= 583 lbs./day | 27,000 X 0.35 ÷ 100 = 94.5 lbs./day |
| Strategic Plan |  |  |  |  |  |  |

II. Nutrients Retained by Animal

| a. Animal Group | h. Maximum One-Time Capacity (# of animals) | i. Average Daily Gain / Animal (lbs./day) (see Form 2) | Live Weight Nutrient Concentration |  | Daily Nutrients Retained by Animal (lbs/day) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | j. N | k. P | l. Nitrogen (h x i x j) | m. Phosphorus (h x i x k) |
| Example: Beef | 1,000 | 4.1 lbs./d | 0.026 | 0.0070 | 1,000 X 4.1 X 0.026 = 107 lbs./d | 1,000 X 4.1 X 0.0070 = 28.7 lbs./d |
| Beef |  |  | 0.026 | 0.0070 |  |  |
| Dairy |  |  | 0.027 | 0.0075 |  |  |
| Pork |  |  | 0.024 | 0.0050 |  |  |
| Hens |  |  | 0.028 | 0.0058 |  |  |
| Broilers |  |  | 0.028 | 0.0058 |  |  |
| Turkeys |  |  | 0.028 | 0.0059 |  |  |

III. Nutrients Retained by Animal Products

| a. Animal Product | n. Daily Production (lbs./day) (see Form 3) | Nutrient Concentration of Animal Products |  | Nutrients Retained by Animal Products (lbs./day) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | o. Nitrogen (lbs. N / lb. prod.) | p. Phosphorus (lbs. P / lb. prod.) | q. Nitrogen (n x o) | r. Phosphorus ( n x p) |
| Milk1 |  | 0.0050 | 0.0010 |  |  |
| Eggs1 |  | 0.0166 | 0.0021 |  |  |

IV. Nutrient Excretion by Livestock

| a. Animal Group/Product | s. Days Fed per Year (days / yr.) (Forms 2 or 3) | Annual Nutrient Excretion |  |  |
| --- | --- | --- | --- | --- |
|  |  | t. N [s x ( f – l )] or [s x ( f – q )] (lbs./yr.) | u. P [s x ( g – m )] or [s x ( g – r )] (lbs./yr.) | v. P2O52 (u x 2.3) (lbs./yr.) |
| Example: Beef Finisher | 350 days/year | 350 x (583 – 107) = 167,000 lbs /year | 350 x (94.5 – 28.7) = 23,000 lbs./year | 23,000 x 2.3 =. 53,000 lbs P2O5 / year |
| Total |  | w. |  | x. |

- Nitrogen in feed = Protein ÷ 6.25,  N in milk = Protein ÷ 6.38.  Nitrogen content of 0.0050 assumes 3.2% protein in milk,  
Nitrogen in eggs =    Protein ÷ 6.25.  Nitrogen content of 0.0166 assumes 10.4% protein in eggs.

2.	Lbs. P2O5  =  lbs. P  X  2.3

24

Instructions for Nutrients Available

after Storage Losses (Form 8)

### Purpose

Form 8 estimates the quantity of nutrients remaining after losses from manure storage.

References

NRCS Agricultural Waste Management Field Handbook (Chapter 11), available online at:

http://www.ncg.nrcs.usda.gov/awmfh.html

Manure Nutrient Inventory Spreadsheet, available online at: http://cnmp.unl.edu/cnmpsoftware.html, completes these same calculations in a Microsoft® Excel spreadsheet.

### Instructions

Nitrogen and phosphorus is lost during the storage or treatment phases of manure handling.  Form 8 allows one to develop a “ballpark” estimate of the nutrients remaining after storage losses.  Producers must identify the manure storage or treatment system that most closely approximates their manure management facility, transfer the nutrient production numbers from Form 6 or 7, and complete the appropriate calculations.

- Regularly harvested manure (Part a of Form 8).

Step 1.  Identify the manure storage system(s) used in this livestock operation from the listing in the left hand column (Column a).

Step 2. Enter required data for each manure storage system.  The user must enter the following values:

- Col. b:  Total nitrogen excretion by livestock from Form 6 (Column n) or Form 7 (Column t).

- Col. e:  Total phosphorus (as P2O5) excretion by livestock from Form 6 (Column i) or Form 7 (Column v).

Step 3.  Complete the following calculations.

- Col. d:  Available nitrogen after losses   =  Col. b  x  Col. c

- Col. g:  Available phosphorus (as P2O5) after losses  =  Col. e  x  Col. f

Note: The multiplication factor is the portion of nutrients retained in the manure.  Most lost nitrogen volatilizes into the air as ammonia, and lost phosphorus settles as solids in the lagoon bottom or is lost as runoff from an open lot.  Actual losses from individual situations may vary substantially.

B. Phosphorus retained in settled solids by an anaerobic treatment lagoon (Part B of Form 8)

Step 4. Retained phosphorus in anaerobic lagoon.

- Col. h: Enter the quantity of total manure phosphorus excretion from Form 6 (Column i) or Form 7 (Column v).

- Col. i:  Enter the interval (years) between when settled solids are removed.

- Col. k: Calculate the total phosphorus retained in the settled solids  = Col. h  x  Col. i  x  Col. j.

Form 8. Nutrients Available after Storage Losses

25

A. Regularly harvested manure.

| a. Manure Storage or Treatment System | Nitrogen |  |  | P2O5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | b. Total N Excretion (Form 6 or 7) (lbs.N/yr) | c. Fraction of N Remaining After Storage Loss | d. Available N After Losses (lbs.N/yr.) | e. Total P2O5 Excretion (Form 6 or 7) (lbs.P2O5/yr.) | f. Fraction of P Remaining After Storage Loss | g. Available P2O5 After Losses (lbs.P2O5/yr.) |
| Example: 1. Storage (liquid manure, top loaded storage) | 45,000 (from Form 6) | X 0.70 = | 31,500 | 39,000 | X 1.0 = | 39,000 |
| Strategic Plan 1. Open lot or feedlot (scraped manure from pens and settling basin) |  | X 0.6 = |  |  | X 0.95 = |  |
| 2. Open lot or feedlot (runoff collected in holding pond) |  | X 0.05 = |  |  | X 0.05 = |  |
| 3. Manure pack under roof |  | X 0.70 = |  |  | X 1.0 = |  |
| 4. Bedded pack for swine. (e.g. hoop building) 1 |  | X 0.50 = |  |  | X 1.0 = |  |
| 5. Bedded pack & compost for swine. (e.g. hoop building) 1 |  | X 0.35 = |  |  | X 1.0 = |  |
| 6. Solid/semi-solid manure & bedding held in roofed storage |  | X 0.75 = |  |  | X 1.0 = |  |
| 7. Solid/semi-solid manure & bedding held in unroofed storage |  | X 0.65 = |  |  | X 0.95 = |  |
| 8. Liquid/slurry storage in covered storage |  | X 0.90 = |  |  | X 1.0 = |  |
| 9. Liquid/slurry storage in uncovered storage |  | X 0.75 = |  |  | X 1.0 = |  |
| 10. Storage (pit beneath slatted floor) |  | X 0.85 = |  |  | X 1.0 = |  |
| 11. Poultry manure stored in pit beneath slatted floor |  | X 0.85 = |  |  | X 1.0 = |  |
| 12. Poultry manure on shavings or sawdust held in housing |  | X 0.70 = |  |  | X 1.0 = |  |
| 13. Compost |  | X 0.70 = |  |  | X 0.95 = |  |
| 14. 1-Cell anaerobic treatment lagoon |  | X 0.20 = |  |  | X 0.35 = |  |
| 15. Multi-cell anaerobic treatment lagoon1 |  | X 0.10 = |  |  | X 0.35 = |  |
| 16. Other: |  | X = |  |  | X = |  |

B. Phosphorus (as phosphate) retained in settled solids by an anaerobic treatment lagoon1

| h. Total P2O5 Excreted Annually (from Form 6 or Form 7) | i. Years Between Solids Removal | j. Portion Retained in Settled Solids | k. Total P2O5 in settled solids (h x i x j) |
| --- | --- | --- | --- |
| lbs./ac. | x | x 0.65 | = |

1.	This applies to an anaerobic treatment lagoon with a permanent liquid pool and no agitation of contents or pumping from lagoon bottom during time of application.

Instructions for Manure Nitrogen after

26

Land Application Losses (Form 9)

### Purpose

The purpose of Form 9 is to estimate the quantity of nutrients remaining after losses during manure application.  For nitrogen, this procedure estimates ammonia losses but assumes all organic nitrogen will be crop available.  This assumption will produce an over-estimate of crop available nitrogen for a single year, but provide a high estimate of land needed for management of manure nitrogen.

References

NRCS Agricultural Waste Management Field Handbook (Chapter 11), available online at:

http://www.ncg.nrcs.usda.gov/awmfh.html

Manure Nutrient Inventory Spreadsheet, available online at: http://cnmp.unl.edu/cnmpsoftware.html, completes these same calculations in a Microsoft® Excel spreadsheet.

### Instructions

Ammonia volatilization losses during land application are estimated in Form 9 and are deducted from total available nitrogen after storage losses.  The magnitude of ammonia losses depends upon the application method.  The outcome of Form 9 is the total available manure nitrogen that must be managed in crop production systems.

Col. a: Identify a manure storage and treatment system used on this livestock operation from the listing in Form 8 (Column a)  for which available nutrient calculations were completed.  Space is available for identifying three manure storage and treatment systems.

Col. b: Record the available nitrogen after manure storage losses from Form 8, Column d, into Column b of Form 9.

Col. c:  Identify the application method that most closely approximates the methods used on your farm (see Column h of Form 9 for choices and enter selection into Column c).  More than one application method may be used for the manure from a single manure storage and treatment system.

Col. d:  Fraction applied by this method. For example, you may broadcast one-third of your manure without incorporation and broadcast and incorporate the remaining two-thirds within one day.  Enter it in decimal form.

Col. e:  Record the fraction of nitrogen remaining after land application (from Column i) for the selected application methods.

Col. f:  Available nitrogen after land application losses   =  Column b  x  Column d  x  Column e

Cell g:  Sum the total nitrogen available by summing values in Column f and recording the total in Cell g.

27

Form 9. Nitrogen Available After Land Application Losses

| Manure Storage or Treatment System |  | Application Method |  |  | f. Nitrogen Available after Storage and Land Application Losses (Cols. b x d x e) (lbs. N/year) |
| --- | --- | --- | --- | --- | --- |
| a. Reference Number from Form 8, Col. a | b. Manure N after Storage System Losses (Form 8, Col. d) (lbs. N/year) | c. Application Method (From Col. h below) | d. Fraction of Manure Applied by this Application Method | e. Fraction of N Remaining after Land Application (from Col. i below) |  |
| Strategic Plan Example: 1. Storage (liquid manure, top loaded storage) | 31,500 lbs./year | Broadcast/not incorporated | 0.33 (1/3) | 0.50 | 5,200 lbs. N / yr |
|  | 31,500 lbs./year | Broadcast/incorporated in 24 hrs | 0.67 (2/3) | 0.75 | 15,800 lbs. N / yr |
| TOTAL Example |  |  |  |  | g. lbs N/yr 21,000 lbs N/yr |

The fraction of manure nitrogen after storage losses still potentially available in the soil is estimated below.  Ammonia volatilization causes the predicted losses.  All organic-N is assumed available.

| h. Application Method | i. Fraction Remaining of Manure N after Storage Losses |  |
| --- | --- | --- |
|  | Slurry or Fresh Solids1 | Bedded Pack, Open Lot, or Composted Solids2 |
| #### Injection | 0.95 |  |
| #### Irrigation | 0.75 |  |
| Broadcast and incorporated: Immediately Within 1 day Within 4 days Within 7 days or more Not incorporated | 1.00 0.75 0.60 0.50 0.50 | 1.00 0.90 0.80 0.75 0.75 |

1Assumes manure nitrogen is 50% ammonium and 50% organic-N.

2Assumes manure nitrogen is 25% ammonium and 75% organic-N.

28

Instructions for Land Requirements for

Manure Nitrogen Utilization  (Form 10)

### Purpose

The purpose of Form 10 is to estimate if sufficient land is available for utilizing the nitrogen in manure at agronomic rates.  If there is manure nitrogen (N) remaining, Form 13 (Plan for Manure Transfer to Off-Farm Users) or Form 14 (Land Application Site Agreement) will need to be completed to account for  unused nitrogen.

THIS PROCESS IS NOT INTENDED FOR MAKING CROP NUTRIENT APPLICATION RECOMMENDATIONS OR DEVELOPING AN ANNUAL NUTRIENT MANAGEMENT PLAN.

Regulations

The state of Nebraska requires that a livestock operation demonstrate access to sufficient land to manage the nitrogen in manure.  Although land requirements are not directly related to phosphorus in manure, Nebraska regulations trigger an NDEQ review of land application sites when soil phosphorus levels exceed 150 ppm measured by Bray 1 or Mehlich III soil tests (or 100 ppm by Olsen soil test).  To avoid this trigger, sufficient land is critical for managing the phosphorus in manure.

References

NRCS Agricultural Waste Management Field Handbook (Chapter 11), available online at:

http://www.ncg.nrcs.usda.gov/awmfh.html

Manure Nutrient Inventory Spreadsheet,  available online at: http://cnmp.unl.edu/cnmpsoftware.html, completes these same calculations in a Microsoft Excel spreadsheet.

### Instructions

# Nitrogen utilization by cropping systems is compared to the  manure nitrogen after losses to determine if sufficient land is available for agronomic application of manure nitrogen. Typical cropping programs, yields, and crop nitrogen requirements are entered for individual fields available for land application.  The nutrient requirements of individual fields are subtracted from the previous estimates of manure nitrogen.  This process is repeated until a sufficient number of fields are identified to utilize all manure nitrogen.

Step 1.  Enter required information including:

Cell a:  Total available manure nitrogen from Form 9, Cell g.

Col. b:  Field or management area ID for fields to be used for manure application.  Names should match those included in the Inventory Section (Form 4).

Col. c:  Size of individual fields in acres. (This should also match Form 4.)

Col. d: Crop grown for a typical year.  Acres for individual crops should match a typical rotation.  For example, a 200-acre farm with a corn-soybean rotation should list fields with a total of 100 acres of corn and 100 acres of soybeans.

Col. e:  Expected yield.  Enter the five-year historical average (excluding years with unusual stress) plus 5 percent.  Units: bu./ac., tons/ac., lb./ac.

Col. f:  Crop Nitrogen Requirement. (approximation for long-term planning or permit application only) =

Yield Goal       X        N Utilization Factor       X        N Removal Rate (from Reference Table R-1)

Use a N utilization factor of 1.3 for corn and small grains; 0.5 for legumes; and 1.0 for grain sorghum, all other forage, silage and other crops. This approximation of crop nitrogen requirement assumes a moderately low soil residual nitrate level of 3 to 6 ppm and average soil organic matter level. If nitrogen rate recommendations based on past soil tests are available for an individual field, that value may be a preferred alternative to this estimate.

Col. g:  Sum of all non-manure nitrogen credits not accounted for in the value in Col. f (e.g. legumes [Table R-3], residual soil nitrate, irrigation water nitrates) and planned commercial fertilizer application.

Step 2.  Complete the following calculations.

Col. h:  Manure nitrogen requirement per acre  =  Column f  -  Column g

Col. i:   Manure nitrogen use by field  =  Column c  x  Column h

Col. j:  Remaining manure nitrogen to be used by other fields  =  Cell a  -  Column i   (for first field entry)

For all remaining fields,  Column j  =  Column j for previous field  -  Column i for current field.

Step 3.  Enter additional fields and repeat calculations until Remaining Manure Nitrogen (Column j) is 0.  If some manure nitrogen is left over after all available fields have been utilized, then either arrange for additional land area for manure application or use Form 13 to show your plan to transfer manure nutrients to off-farm customers.

29

Form 10. Land Requirements for Manure Nitrogen Utilization

# Caution:  This simplified estimating procedure does not account for crop availability or manure organic nitrogen, various nitrogen credits (e.g. residual nitrates in the soil), or inefficiencies in crop recovery of nitrogen.  As such this procedure is intended only to provide NDEQ an estimate of “approximate” land required for managing manure nitrogen as part of a permit application.  It should not be used for estimating actual manure application rates.

## Total Manure Nitrogen After Losses (Form 9, Cell g) =                        (a)  lbs. N/yr.

Example: 21,000 lb.N/yr.

| Strategic Plan b. Field ID | c. Acres | d. Crop | e. Expected Yield units |  | f. Crop Nitrogen Requirement (lbs. N/ac.) 1 | g. Nitrogen Credits (lbs. N/ac.) | h. Manure-N Requirement (f – g) (lbs. N/ac.) | i. Manure-N Use by Field (c X h) (lbs. N/field) | j. Remaining Manure N (lbs/yr) …. [ (a - i) or (last j - i)] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example1 | 80 | Corn | 170 | bu/ac. | 166 lb./acre | 30 lb/ac. | 136 lb./ac | 10,900 lbs | 10,100 lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |

1For the example, crop N removal equals 170 bu./ac. x 1.3 x 0.75 lb. N/bu. (from Reference Table R-1) or 166 lbs. of  N/acre. A nitrogen utilization efficiency factor of 1.3 is used. Good nitrogen management practices would be necessary to achieve this efficiency. This is an approximation for long-term planning or permit applications when soil test results would not be available.

Good

Steward

Instructions for Land Requirements for

30

Manure Phosphorus Utilization  (Form 11)

### Purpose

The purpose of Form 11 is to estimate if sufficient land is available for utilizing the phosphorus in manure at agronomic rates.  This estimate is currently not required in Nebraska; however, proposed U.S. Environmental Protection Agency rules may soon require that manure application be based on phosphorus.

THIS PROCESS IS NOT INTENDED FOR MAKING CROP NUTRIENT APPLICATION RECOMMENDATIONS OR DEVELOPING AN ANNUAL NUTRIENT MANAGEMENT PLAN.

References

NRCS Agricultural Waste Management Field Handbook (Chapter 11), available online at:

http://www.ncg.nrcs.usda.gov/awmfh.html

Manure Nutrient Inventory Spreadsheet, available online at:  http://cnmp.unl.edu/cnmpsoftware.html,

completes these same calculations in a Microsoft® Excel spreadsheet.

### Instructions

# Phosphorus utilization by cropping systems is compared to the available manure nutrients after losses to determine if sufficient land is available for agronomic application of manure phosphorus. Typical cropping programs, yields, and crop phosphorus requirements are entered for individual fields available for land application.  The nutrient requirements of an individual field are subtracted from the previous estimates of available manure phosphorus.  This process is repeated until sufficient fields are identified to utilize all manure phosphorus.

Step 1.  Enter required information including:

Cell a:  Total available manure phosphorus from Form 8, Col. g.  Sum all Column g entries.

Col. b:  Field or management area ID for fields to be used for manure application.  Names should match those included in the Inventory section (Form 4).

Col. c:  Size of individual fields in acres (this should also match Form 4).

Col. d:	Crop grown for a typical year.  Acres for individual crops should match a typical rotation.

Col. e:  Expected yield.  Enter the five-year historical average (excluding years with unusual stress) plus 5 percent.  Units: bu./ac., tons/ac., lb./ac.

Col. f:	Crop Phosphorus Requirement. This should be based upon historical soil tests and recommendations.  Look these up in Reference Tables R-18 to R-32.  If this information is not available, an alternative is to use crop phosphorus removal rates from Reference Table R-1.

Col. g:  Sum of all non-manure phosphorus credits or planned commercial fertilizer application.

Step 2.  Complete the following calculation.

Col. h:  Manure phosphorus requirement per acre  =  Column f  -  Column g

Col. i:   Manure phosphorus use by field  =  Column c  x  Column h

Col. j: 	Remaining manure phosphorus to be used by other fields  =  Cell a  -  Column i   (for first field entry)

For all remaining fields,  Column j  =  Column j for previous field  -  Column i for current field.

Step 3.  Enter additional fields and repeat calculations until Remaining Manure Phosphorus (Column j) is 0.  If some manure phosphorus is left over after all available fields have been utilized, then either arrange for additional land area for manure application, or use Form 13 to show your plan to transfer manure nutrients to off-farm customers.

Good

Steward

31

Form 11. Land Requirements for Manure Phosphorus Utilization

| ## Available Manure Phosphorus (Form 8, Col. g) = | ## (a) | ## lbs. P2O5 |
| --- | --- | --- |
|  | ## Example: 39,000 lb./year |  |

| b. Field ID | c. Acres | d. Crop | e. Expected Yield (units) |  | f. Crop P2O5 Requirement1 (lbs./acre) | g. P2O5 Credits (lbs./ac.) | h. Manure P2O5 Requirement (f – g) (lbs./acre) | i. Manure P2O5 Use by Field (c x h) (lbs./field) | j. Remaining Manure P2O5 (a-i) (lbs./year) (j above - i) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example | 80 | Corn | 170 | bu./ac | 51 lbs./ac | 0 lbs./ac | 51 lbs./ac | 4,100 lbs. | 34,900 lbs. |
| Strategic Plan |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |
|  |  |  |  |  | lbs/ac | lbs/ac | lbs/ac | lbs. | lbs. |

1For the corn example, crop phosphorus removal equals 170 bu./ac. X 0.30 lb. P2O5 / bu.  (from Reference Table R-1) or 61 lbs.

of P2O5/ac.

Good

Steward

32

Instructions for Land Requirements for Sludge P

Utilization from Anaerobic Lagoon (Form 12)

### Purpose

The purpose of Form 12 is to estimate if sufficient land is available for utilizing the phosphorus in sludge harvested from anaerobic lagoons if applied at phosphorus-based rates.  This worksheet makes the assumption that all sludge will be harvested which may not be true for lagoon management.  An understanding of the land requirements for utilizing anaerobic lagoon phosphorus (the only nutrient that can be predicted with some accuracy in advance of sludge removal) is important to NDEQ’s required Sludge and Sediment Management Plan.

THIS PROCESS IS NOT INTENDED FOR MAKING CROP NUTRIENT APPLICATION RECOMMENDATIONS OR DEVELOPING AN ANNUAL NUTRIENT MANAGEMENT PLAN.

Regulations

The state of Nebraska requires that a permitted livestock operation have a Sludge and Sediment Management Plan.  A discussion of measures taken to utilize sludge and sediment nutrients at agronomic rates may be included with this plan.

References

NRCS Agricultural Waste Management Field Handbook (Chapter 11), available online at:

http://www.ncg.nrcs.usda.gov/awmfh.html

Manure Nutrient Inventory Spreadsheet,  available online at: http://cnmp.unl.edu/cnmpsoftware.html, completes these same calculations in a Microsoft® Excel spreadsheet.

### Instructions

# Phosphorus utilization by cropping systems is compared to the available sludge nutrients after losses to determine if sufficient land is available for agronomic application of sludge phosphorus.

Step 1.  Enter required information including:

Cell a:	Total available sludge phosphorus.  Obtain value from Form 8, Column k.

Col. b:		Field ID for fields to be used for sludge application.  Names should match those included in the Inventory Section (Form 4).  These fields should be different from those fields that have historically received lagoon effluent or manure if those nutrient sources are meeting crop nutrient needs.

Col. c:	Size of individual fields in acres. (This should also match Form 4).

Col. d:	Crop grown for a typical year.

Col. e:	Expected yield.  Expected yield should be estimated based upon a five-year historical average plus 5 percent.

Col. f:	Crop phosphorus requirements. If this information is not available from Reference Tables R-18 to R-32, use crop phosphorus removal rates from Reference Table R-1 (see page 78).

Col. g:	Years of crop phosphorus needs to be supplied by one sludge application. If soil phosphorus levels are less than 50 ppm Bray 1 or 35 ppm Olsen, it may be acceptable to build soil phosphorus levels by applying sufficient phosphorus to supply several years of crop needs (maximum of five years).  In addition, a final application rate should compare sludge nitrogen and crop nitrogen requirements (determined by laboratory analysis) to avoid over-application of nitrogen for a single cropping season.

Step 2.  Complete the following calculation.

Col. h:	Sludge phosphorus use by field  =  Column c  x  Column f  x  Column g.

Col. i:	Remaining sludge phosphorus to be applied to other fields  =  Cell a  -  Column h   (for first field entry)

For all remaining fields:  Column i  =  Column i for previous field  -  Column h for current field

Step 3.  Enter additional fields and repeat calculations until Remaining Sludge Phosphorus (Column i) is 0.  If some manure phosphorus is left over after all available fields have been utilized, then either arrange for additional land area for manure application, or use Form 13 to show your plan to transfer manure nutrients to off-farm customers.

Good

Steward

33

Form 12. Land Requirements for Using Phosphorus in

# Sludge from Anaerobic Lagoons

## Available Sludge Phosphorus (Form 8, Col. k) =	     	  (a)    lbs. P2O5

Example: 164,000 lbs

Sludge was accumulated over       years (Form 8, Col. i).

| Strategic Plan b. Field ID | c. Acres | d. Crop | e. Expected Yield (units) |  | f. Crop P2O5 Requirement1 (lbs./acre) | g. Years of Crop P2O5 Needs Supplied2 | h. Sludge- P2O5 Use by Field (c x f x g) (lbs.) | i. Remaining Sludge- P2O5 (a - h) (lbs.) (i above - h) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example | 160 | Corn | 170 | bu/ac. | 51 lbs./acre | 5 yrs. | 41,000 lbs. | 123,000 lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |
|  |  |  |  |  | lbs/acre | yrs. | lbs. | lbs. |

1For the corn example, crop phosphorus removal equals 170 bu./ac. times 0.3 lb. P2O5 /bu. (from Reference Table R-1) or 61

lbs. of P2O5/acre.

2A final application rate should compare nitrogen applied as sludge and crop nitrogen requirements to avoid overapplication

of nitrogen for any single cropping season.

34

Instructions for Manure Transfer

to Off-Farm Users (Form 13)

### Purpose

This form will guide the development of a management plan for transferring or marketing manure to off-farm users acceptable to NDEQ’s requirements for utilizing manure nitrogen.

Regulations

The state of Nebraska requires that a livestock operation demonstrate access to sufficient land to manage the nitrogen in manure.  If sufficient land is not available for utilization of manure nutrients on crop land owned or managed by the livestock manager, then two alternatives exist for documenting environmentally sound management of the manure nutrients that will need to be transferred to off-farm users:

- A plan for marketing or transferring manure to off-farm users of manure (Form 13).  This option may be acceptable to NDEQ in some situations where a livestock operation is marketing manure as a fertilizer.  This marketing plan will need to be supplemented with records documenting the movement of sufficient manure to off-farm users to meet the goals of the plan.  Copies of sales receipts and records of sales will be necessary.

- A signed land use agreement (Form 14) describing the agreement between the landowner and the livestock producer for manure application to that land (preferred by NDEQ).

### Instructions

Step 1.  Enter required information including:

Col. a:  Annual quantity of manure to be transferred to crop producers, horticulture related businesses, and other consumers you identify.

Col. b & Col. d:  Expected concentration of manure nitrogen, and manure phosphorus (optional). Terms: lbs./ton, lbs./1000 gallons, or lbs./acre-inch.

Row f:  Enter the anticipated excess manure nitrogen following manure applications to owned or rented land, or land receiving manure based on easements.  Obtain estimate from lowest remaining nitrogen value from Form 10, Column j.

Step 2.  Complete the following calculation.

Col. c:  Total manure nitrogen transferred off-farm annually  =  Column a  x  Column b.

Col. e:	Total manure phosphorus transferred off-farm annually  =  Column a  x  Column d.

Row h: Unused manure nitrogen  = Row f  -  Row g.  Answer in Row h should be near 0.  If unused manure nitrogen is significant, additional land for application or additional manure sales will need to be identified.

Step 3.  Identify and describe the services the livestock operation will provide in support of manure transfer

or sale.

Step 4.  Identify frequency with which summary of actual manure sales or transfer will be reported to NDEQ

and attach copy of record keeping forms to be maintained.

Step 5. Prepare a business plan, if appropriate, for marketing of manure and attach to report.

35

Form 13.  Plan for Manure Transfer

to Off-Farm Users

Information in Columns a-c is  required for a state operating permit.

1. Manure nutrients to be transferred to off-farm users.

|  | a. Annual Quantity of Manure Transferred | Nitrogen Transferred Off-Farm |  | Phosphorus Transferred Off-Farm |  |
| --- | --- | --- | --- | --- | --- |
|  |  | b. Concentration From Manure Analysis | c. Total Annual Transfer (lbs.) [a x b] | d. Concentration From Manure Analysis | e. Total Annual Transfer (lbs.) [a x d] |
| f. Excess Manure Nitrogen (Form 10, Column j) |  |  |  |  |  |
| Strategic Plan Off-Farm Users: Crop Producers | Tons Gallons Acre-in |  |  |  |  |
| Horticulture Businesses | Tons Gallons Acre-in |  |  |  |  |
| Others: | Tons Gallons Acre-in |  |  |  |  |
| g. Total Off-Farm Uses of Manure | Tons Gallons Acre-in |  |  |  |  |
| h. Unused Manure Nitrogen [Row f – Row g] |  |  |  |  |  |

2. Services to be offered in support of manure transfer to off-farm users.

a.	Handling and application services to be included with manure.

Loading of manure	 Transport of manure to site

Land application of manure	 Incorporation of manure

|  | Describe: |  |
| --- | --- | --- |

- Agronomic services to be included for assisting users in agronomic applications of manure.

Calibration of manure applicator	 Soil testing

Manure analysis	 Consulting agronomist

Customer report following application of manure nutrient rates

|  | Describe: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| c. Nuisance avoidance services to be included with transfer of manure to off-farm user: |  |  |  |  |  |
| Incorporation of manure |  |  | Setback of application from homes:___________ |  |  |
| Composting of manure |  |  | Setback of application from waters of the state: |  |  |
| Composting |  |  |  |  |  |
| Notification of neighbors |  |  | Other setbacks: |  |  |
| Notification of local government |  |  | Timing to limit nuisance: |  |  |
| 3. Attach copy of recording forms to be used. |  |  |  |  |  |

4. Manure marketing plan.  Attach copy of a) past years’ manure sales or transfers or b) plan for promotion, pricing, and other business issues for transferring manure to off-farm users.

Instructions for

36

Land Application Site Agreements (Form 14)

#### Purpose

A signed agreement is required for manure application on land not owned or managed by the animal feeding operation.  This form provides a sample agreement for land application of manure on land not owned by the animal feeding operation.  If a lease agreement includes terms for manure application, a copy of this lease agreement is an acceptable alternative.

NDEQ Title 130

The state of Nebraska requires that a livestock operation demonstrate access to sufficient land to manage the nitrogen in manure.  If sufficient land is not available for utilization of manure nutrients on cropland owned or managed by the livestock manager, two alternatives exist for documenting environmentally sound management of the manure nutrients that will need to be transferred to off-farm users:

- A plan for marketing or transferring manure to off-farm users of manure (Form 13).  This option may be acceptable to NDEQ in some situations where a livestock operation is marketing manure as a fertilizer.  This marketing plan will need to be supplemented with records documenting the movement of sufficient manure to off-farm users to meet the goals of the plan.  Copies of sales receipts and records of sales will be necessary.

- A signed land use agreement (Form 14) describing the agreement between the landowner and the livestock producer for manure application to that land (preferred by NDEQ).

Sample Forms Provided

1.	Form 14.  Agreement for land application of manure.

37

###### Form 14. Land Application Site Agreement

###### for Manure Application

| I, |  |  |  |  |  |  | , hereby give permission to the animal production facility |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| owned by |  |  |  |  |  |  |  |  |  | for the application |  |  |
| of animal manure to |  |  |  | acres of my land for the duration of the agreed upon time shown below. The |  |  |  |  |  |  |  |  |
| land involved in this agreement is located at: |  |  |  |  |  |  |  |  |  |  |  |  |
| 1/4 | Sec-tion | Town- ship | Range |  | E or W | County |  | Crop Acres | Crop Rotation |  | Average Yields |  |
| Strategic Plan |  |  |  |  | or |  |  |  |  |  |  |  |
|  |  |  |  |  | or |  |  |  |  |  |  |  |
|  |  |  |  |  | or |  |  |  |  |  |  |  |
|  |  |  |  |  | or |  |  |  |  |  |  |  |

Restrictions agreed upon by all parties include:

| Responsibilities of individual parties include1: |  | Landowner | Animal Producer | Other: |
| --- | --- | --- | --- | --- |
|  | Crop nutrient management plan………... |  |  |  |
|  | Soil testing*……………………………... |  |  |  |
|  | Manure analysis………………………... |  |  |  |
|  | Manure applicator calibration……….…. |  |  |  |
|  | Crop nutrient status monitoring………... |  |  |  |
|  | Record keeping* ………………………... |  |  |  |

*  Copies must be maintained by livestock facility owner.

I understand that this manure contains organic matter, nitrogen, phosphorus, potassium, trace elements and pathogens which, if applied at agronomic rates at appropriate times to minimize surface water runoff, should not harm my land, my crops, or waters of the state of Nebraska.  I also understand that the use of animal manure will reduce my need for commercial fertilizer, that a nutrient management plan that credits manure nutrients should be implemented, and that failure to reduce commercial fertilizer use when animal manure is applied is likely to have a detrimental impact on water quality.

| Period of Agreement: | From: / / month day |  | 20____ year |  | To: ______/______ / month day |  |  |  | 20____ year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Landowner Signature |  |  |  |  |  | Date: |  |  |  |
| ##### Animal Facility Owner Signature |  |  |  |  |  |  |  |  |  |

|  | Landowner |  | Animal Facility Owner |  |
| --- | --- | --- | --- | --- |
| Name: |  |  |  |  |
| Address: |  |  |  |  |
| Phone Number: |  |  |  |  |

1. Assigned responsibilities should match with Nutrient Management Activities Plan (Form 16).

Instructions for

38

Emergency Response Plan (Form 15)

#### Purpose

Form 15 provides a generic Emergency Response Plan from which a plan can be developed to address possible emergency situations specific to an individual farm that may lead to a manure spill or discharge. Fill out one copy of this form for each potential emergency situation related to the land application manure that applies to your livestock operation.

NDEQ Title 130

Permitted facilities must maintain an Emergency Response Plan for controlling a spill or discharge of animal waste due to any number of causes including:

1.	power failure

2.	storms or extended wet periods

3.	accidents

4.	equipment failure (including irrigation equipment and components)

5.	failure of components of the livestock waste control facility

Permitted facilities also are required to report any discharge to NDEQ within 24 hours of the event and file a written report with NDEQ within seven days of the discharge.  The department may request rainfall, land application, and storage system records for up to 12 months prior to the discharge.

Instructions

- Identify situations associated with land application of manure or other effluent that may lead to a spill or discharge:

Overturned manure spreader or slurry tank or other spillage of load

Broken transfer pipe or connection

Pivot that becomes stuck or immobile while continuing to pump effluent

Manure runoff from land application site

Other:     	*

Other:     	*

*  Emergency response plans necessary for manure storage related emergencies should be prepared as part of the “Manure and Open Lot Runoff Storage Workbook.”

- For each issue identified in the attached generic Emergency Response Plan, identify the information important to your employees for responding to an emergency situation.  This report is intended for emergency situations that lead to manure or other effluent spills.  It also can be used for other emergency situations such as fire, loss of power, storm damage, etc.

39

Form 15.  Emergency Response Plan

Date:

| Farm Name and Location: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Potential Cause of Discharge. Possible situations which may require an Emergency Response Plan: |  |  |  |  |  |  |  |  |  |  |  |
| Power failure |  |  |  |  | Storm/extended wet period |  |  |  | Accident |  |  |
| Equipment failure |  |  |  |  | Failure of berm or other facility component: |  |  |  |  |  |  |
| Strategic Plan Describe: |  |  |  |  |  |  |  |  |  |  |  |
| In Case of an Emergency: |  |  |  |  |  |  |  |  |  |  |  |
| - Implement the following first response or containment steps: |  |  |  |  |  |  |  |  |  |  |  |
| - Assess the extent of the emergency and determine required help. Collect the following information: |  |  |  |  |  |  |  |  |  |  |  |
| Description of emergency Quantity of manure/effluenet released Required help |  |  |  |  |  |  |  |  |  |  |  |
| Obvious damage: employee injury, fish kill, property damage Containment actions in progress |  |  |  |  |  |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |  |  |  |
| 3. Contact the farm’s emergency response team leader: |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  | Phone: |  |  |  |  |
| Name: |  |  |  |  |  |  | Phone: |  |  |  |  |
| 4. Give the team leader the following information: a. Your Name b. Spill/emergency location c. Information collected in Step 2 |  |  |  |  |  |  |  |  |  |  |  |
| 5. Available equipment/supplies for responding to emergency: |  |  |  |  |  |  |  |  |  |  |  |
| Equipment/Supplies |  |  |  |  | Contact Person |  |  |  | Phone Number |  |  |
| 6. Contacts to be made by farm’s emergency response team leader (discharge must be reported to NDEQ within 24 hours): |  |  |  |  |  |  |  |  |  |  |  |
| Organization |  |  |  |  | Contact Person |  |  |  | Phone Number |  |  |
| NDEQ |  |  |  |  |  |  |  |  | (402) 471-2186 |  |  |
| County contact (sheriff or other) |  |  |  |  |  |  |  |  |  |  |  |
| 7. Additional containment measures, corrective measures, or property restoration measures. |  |  |  |  |  |  |  |  |  |  |  |

8.  Will written report of accident or spill be submitted to NDEQ?   Yes         No

(Written report must be filed with NDEQ within seven days for manure discharges.)

Instructions for Nutrient

40

Management Activities Plan (Form 16)

Purpose

The purpose of a Nutrient Management Activities Plan is to define procedures to be used for manure application, soil testing and manure analysis. These activities are unlikely to change from one cropping year to the next.  As such, this document only needs to be updated as the need arises.  This planning document should be submitted as part of the Livestock Waste Control Facility permit application to NDEQ.

# The Nutrient Management Activities Plan for Soil Testing summarizes the specific activities related to soil sampling procedures, sampling timing and frequency, and laboratory analysis to be completed.  In addition, other indicators of crop nitrogen status to be used can be identified in this section.

# The Nutrient Management Activities Plan for Manure Nutrient Analysis summarizes the specific activities that will be implemented relative to manure analysis, including sampling procedures, frequency and timing of sampling, and analysis to be completed.

The Nutrient Management Activities Plan for Manure Application  summarizes the specific activities to be implemented relative to manure application, including application equipment calibration, setback distances to be maintained, timing and frequency of manure application, and records to be maintained.

Regulations

NDEQ’s Title 130 requires that a permit application include a nutrient management plan that includes: “waste sampling and analysis procedures, land application soil sampling and analysis procedures, and planned application rates, methods, and frequencies.”  To comply with these regulations, the producer will need to submit an Activities Plan for:

- Soil Sampling (page 41)

- Manure Analysis (page 42)

- Manure Application (page 43)

This Nutrient Management Activities Plan should be submitted to NDEQ at the time of a permit application.

References

Guidelines for Soil Sampling (NebGuide 1000), http://www.ianr.unl.edu/pubs/soil/g1000.htm or contact local NU Cooperative Extension office.

Manure Applicator Calibration.  NebGuide G95-1267, http://www.ianr.unl.edu/pubs/wastemgt/g1267.htm

# Manure Application Calibration Guide, page 87 of this workbook.

Manure Testing: What to Request? Nebfact 02-507, http://www.ianr.unl.edu/pubs/wastemgt/nf507.htm

Manure Characteristics, Midwest Plan Service Publication 18, Section 1, available from your local 
NU Cooperative Extension office.

Nutrient Management for Agronomic Crops in Nebraska, EC155, available to order from your local 
NU Cooperative Extension office.

# Sampling Manures for Nutrient Analysis, NebGuide G02-1450,

# http://www.ianr.unl.edu/pubs/wastemgt/g1450.htm

The Corn Stalk Nitrate Test, NebFact 01-491, http://www.ianr.unl.edu/pubs/fieldcrops/nf491.htm

Using a Chlorophyll Meter to Improve Nitrogen Management, NebGuide G1771, http://www.ianr.unl.edu/pubs/soil/g1171.htm

University of Nebraska Soils Home Study Course, http://www.ianr.unl.edu/soilshomestudy/index.htm or call  (402) 821-2151.

Form 16.  Nutrient Management Activities Plan

41

# for Soil Testing

Describe in Column a the typical procedures used for soil sampling for your most common crop(s).  In Columns b and c, identify variations in the typical procedure that may be used for specialty crops or situations.

|  | a. Typical Procedures for Crop: _________ Approx. acres: | b. Approx. acres: | c. Approx. acres: |
| --- | --- | --- | --- |
| Strategic Plan Frequency and Soil Sampling for N and P |  |  |  |
| Sampling Procedures for N and P: (e.g. no. of cores / area, depth, acres / sample area, etc.) |  |  |  |
| Sample Analysis Procedures for N and P: | Soil nitrate Bray 1 for P Olsen for P Lab: Melich III for P Other: | Soil nitrate Bray 1 Olsen Melich III Other: | Soil nitrate Bray 1 Olsen Melich III Other: |
| Source (e.g. UNL) of Nitrogen and Phosphorus Recommen-dations |  |  |  |

# Additional Crop Nutrient Status Measurements

# (e.g. other soil nitrate tests, irrigation water tests, chlorophyll meter readings, corn stalk nitrate test, etc.).

| Test: |  |  |  |
| --- | --- | --- | --- |

Timing of Selected Activities.  Check appropriate months when practice should occur.

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sept | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil sampling |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional crop nutrient status indicators |  |  |  |  |  |  |  |  |  |  |  |  |

Form 16. Nutrient Management Activities Plan

42

# for Manure Nutrient Analysis

|  | Manure Handling System # : | Manure Handling System # : |
| --- | --- | --- |
| Manure Sampling Frequency: |  |  |
| Sample Collection Procedures: (see publications No. 4 and 7, pg. 40) |  |  |
| Analysis to be Completed | Total nitrogen (required) Ammonium nitrogen (required) Organic nitrogen (recommended) Phosphorus (recommended) Potassium Trace minerals Moisture or solids content (recommended) pH Electrical conductivity (recommended*) Other: | Total nitrogen (required) Ammonium nitrogen (required) Organic nitrogen (recommended) Phosphorus (recommended) Potassium Trace minerals Moisture or solids content (recommended) pH Electrical conductivity (recommended*) Other: |
| Other: (e.g. laboratory used) |  |  |

# * 	Recommended for anaerobic lagoons. Conductivity levels less than 7 mmho/cm or 7,000 mho/cm are important to bacterial activity in a well functioning anaerobic lagoon.

Timing of Selected Activities.  Check appropriate months when practice should occur.

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sept | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manure sampling |  |  |  |  |  |  |  |  |  |  |  |  |

Form 16. Nutrient Management Activities Plan

43

# for Manure Application

|  | Manure Handling System # : |  | Manure Handling System # : |
| --- | --- | --- | --- |
| Strategic Plan Priorities for Fields Receiving Manure (response refers to both manure handling systems). | Identify top one or more priorities (number in order) from list below: Potential to reduce commercial Field’s soil loss or runoff potential fertilizer use Field’s odor potential Crops with high nitrogen requirement Field’s current phosphorus level Crops with high phosphorus requirement Fields benefitting from building soil Transport distance organic level Other: |  |  |
| Application Plan: |  |  |  |
| Method of application? |  |  |  |
| Incorporation? |  |  |  |
| Calibration Plan: |  |  |  |
| Method? |  |  |  |
| Frequency? See Applicator Calibration Guide (page 87) |  |  |  |
| Planned Setback Distances: |  |  |  |
| Record Keeping Plan (attach sample forms) | Records will be maintained of current and past: Annual crop plans (required) Actual crop yields (required) Manure analysis (required) Soil tests (required) Manure application rates (required) Off-farm transfers of manure Other: | Records will be maintained of current and past: Annual crop plans (required) Actual crop yields (required) Manure analysis (required) Soil tests (required) Manure application rates (required) Off-farm transfers of manure Other: |  |

Timing of Selected Activities.  Check appropriate months when practice should occur.

|  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sept | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application equipment calibration |  |  |  |  |  |  |  |  |  |  |  |  |
| Application timing |  |  |  |  |  |  |  |  |  |  |  |  |

44

Inventory

Strategic Plan

Annual Plan

Crop Available Manure Nitrogen48

Annual Field Plan for N50

Annual Field Plan for P52

Action Plan54

Review and

Plan Modification

Records

Review as Necessary

Review Annually

NDEQ Permit Application

NDEQ Permit Implementation

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Annual Plan

45

An electronic copy of this workbook and individual worksheets are available online at http://cnmp.unl.edu

46

47

# Annual Plan:

# Overview of Forms 17 to 20

Annual Plan

Purpose

The Annual Plan’s purpose is to:

1.	Balance the available nutrient inputs from all nutrient sources (including manure) with crop requirement for nutrients, and

2.	Insure that adequate nutrients are supplied to sustain economic crop production.

Application of manure and other nutrients at rates matching crop nutrient needs minimizes ground and surface water contamination.  To achieve this balance:

Nitrogen inputs must be matched with crop nitrogen needs for each growing season.  Nitrogen in the nitrate form is very mobile.  Residual nitrate-nitrogen commonly contributes to groundwater contamination.  The goal is to balance nitrogen inputs during the growing season with crop needs and end the growing season with a low residual soil nitrate level.  This is critical for optimum profitability while minimizing the potential for nitrate contamination of groundwater.

Including fertilizer, manure, nitrate

in irrigation water, legume-N  credits.

Crop

Nutrient

Needs

Total Available

Nutrient

Inputs

Phosphorus inputs should match crop phosphorus needs over several growing seasons.  A single phosphorus application at a rate sufficient to meet multiple years’ crop needs (but not exceeding crop nitrogen requirements) is acceptable if future phosphorus applications are reduced or eliminated to allow future crops to utilize excess phosphorus.  Minimizing excess phosphorus accumulation in the soil surface is critical to minimizing phosphorus contamination of surface water.

Regulations

NDEQ’s Title 130 requires that a producer maintain records documenting  procedures used to estimate manure application rate.  To comply with these regulations, the producer should follow these steps:

- Annually update a Crop Available Manure Nitrogen Plan (Form 17) for common manure application rates and the Annual Field Plan for Nitrogen (Form 18).

- From this information, annually prepare an Action Plan (Form 20) detailing manure and fertilizer applications. Keep the plan on file for five years (for NDEQ inspection).

- Maintain an annual set of records documenting the implementation of the Nitrogen Management Plan and Action Plan and keep these records on file for five years (for NDEQ inspection).

References

Determining Crop Available Nutrients from Manure (NebGuide G97-1335).

Nutrient Management for Agronomic Crops in Nebraska (EC 01-155).

University of Nebraska Soils Home Study Course, http://www.ianr.unl.edu/soilshomestudy.

48

##### Nitrogen Management Plan

##### Instructions for Crop Available Manure Nitrogen (Form 17)

Purpose

This worksheet will estimate a crop available nitrogen credit for a known (calibrated) manure application rate. A Manure Use Plan spreadsheet, available at http://cnmp.unl.edu/cnmpsoftware.html, completes these same calculations.

Regulations

NDEQ’s Title 130 requires that manure not be applied “in excess of agronomic rates for nitrogen.”  To comply with these regulations, the producer will need to annually prepare a Nitrogen Management Plan that determines Crop Available Manure Nitrogen, and keep it on file for five years (for NDEQ inspection).

Steps

Col. a:  This “Option #” is used again in the Annual Field Plan (Form 18, Column l) to reference the selected manure application option.

Col. b:  Enter description of manure source (e.g. lagoon, below barn pit, open lot), season of application, and timing of incorporation.

Col. c.  Enter the planned application rate. Application equipment should be calibrated to achieve approximately the desired rate.

Col. d:  Enter the manure’s ammonium-N from lab analysis or, if not available, an approximate nutrient content from Reference Table R-3 (p. 77).

Col. e: Fill in the ammonium-N availability factor based on the most applicable situation from the left box in Figure 1.

Col. f: Calculate crop available ammonium-N (Col. c  x  Col. d x  Col. e)

Col. g: Enter the organic-N in the manure from lab analysis (Total N - Ammonium N). If a manure analysis is not available, see Reference Table R-3.

Col. h: Enter the organic-N availability factor from the middle box in 
Figure 1.

Col. i: Calculate the crop available organic-N (Col. f x Col. g  x  Col. h).

Col. k, l, and m:  Organic-N available over the next three years can be estimated by multiplying the appropriate availability factor in the right hand box of Figure 1 by the value in Col. i.

This procedure should be repeated for each manure application system (or piece of equipment), each application rate, and timing of incorporation.

Figure 1.  Availability factors for manure nitrogen.  Values from figure should be entered into Form 17.

Ammonium-N Available This Year:

Total Manure

Nitrogen

Urine

Organic-N Available This Year2

Beef/Dairy                         	Poultry

Solid (e.g. feedlot)	  0.25 	   Deep pit	0.45

Stored liquid	0.35	   Solid with litter	0.30

Compost	0.15	   Solid without litter	0.35

Swine

Fresh	0.5	Stored liquid	0.35

Next Year:  	0.15

2 Years From Now:	0.07

3 Years From Now:	0.04

Multiply factor by organic-N

content shown in manure analysis

Organic-N Available:

Feces

Sidedress application1: 	Preplant application and incorporated1:

Incorporated	1.0	   Immediately  	                0.95

Sprinkler irrigation	0.5	   One day later  	0.5* 	0.7**

Flood irrigation	0.9	   Two days later	0.25*	0.5**

Preplant application	   	   Three days later  	0.15*	0.35**

Not Incorporated  	0.0	   Seven days or more         0.0**

* Solid Manure          ** Liquid Manure

Ammonium-N

Available This Year

Organic-N

Available This Year

Organic N

From Past Applications

Crop Available

Nitrogen

=

Form 3vailability factor by 0.7.

ing seeded crops such as corn and soybeans. For fall seeded crops such as wheat, multiply org

+9Form 3vailability factor by 0.7.

ing seeded crops such as corn and soybeans. For fall seeded crops such as wheat, multiply org

+

+

1    Incorporation can be accomplished by tillage or by a 0.50 inch or greater rainfall.

2   2	 Organic-N availability assumes spring seeded crops such as corn and soybeans. For winter or spring manure application prior to planting small grains, multiply organic-N availability factor by 0.7.  For late summer or fall manure application prior to planting small grains, use the organic N values shown in Figure 1.

Form 17. Crop Available Manure Nitrogen

| Manure Application Options |  |  |  | Ammonium-N Available This Year |  |  |  | # Organic-N Available This Year |  |  |  | j. This Year’s Total N Avail-able (f + i) (lbs./ac) | Organic-N Available |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a. Option # X X X | b. Manure Source, Season of Application, and Incorporation | c. Planned Application Rate |  | d. Ammonium-N Content (“as is” basis) |  | e. Avail-able Factor (see Figure 1) | f. Available NH4-N (c x d x e) (lbs./ac.) | g. Organic-N Content (“as is” basis) |  | h. Avail-able Factor (see Figure 1) | i. Available Organic-N (c x g x h) (lbs./ac.) |  | k. Next Year (c x g x 0.15) (lbs./ac) | l. 2 Years from Now (c x g x 0.07) (lbs./ac.) | m. 3 years from Now (c x g x 0.04) (lbs./ac) |
| Ex. | Feedlot manure, surface applied, incorporate in 24 hrs. | 18 | tons/ac 1000 gal/ac ac-in/ac | 4 | Lbs./ton Lbs./1000 gal Lbs./ac-in | 0.5 | 36 | 16 | Lbs./ton Lbs./1000 gal Lbs./ac-in | 0.25 | 72 | 108 | 36 | 18 | 9 |
| 1 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 2 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 3 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 4 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 5 |  |  | tons/ac 1000 gal/acac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 6 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 7 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |  |  |  |
| 8 |  |  | tons/ac 1000 gal/ac ac-in/ac |  | tons/ac 1000 gal/ac ac-in/ac |  |  |  | tons/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |

49

Annual Plan

50

##### Instructions for Annual Field Plan for Nitrogen (Form 18)

Purpose

This planning guide will determine the amount of  nitrogen that will be needed (including manure nitrogen) to meet crop nitrogen requirements.  All crop nitrogen annual planning documents are organized to maintain multiple years of records for a single field on one page.  Make additional copies of  Form 18 so that one copy is available for each field or management area receiving manure. A Manure Use Plan spreadsheet, available at http://cnmp.unl/cnmpsoftware.html, completes these same calculations.

Instructions

At the top of each page: Identify the field or management area. Enter the soil test organic matter,  the irrigation water nitrate concentration (if irrigated), and the acre-inches of water usually applied by mid season (e.g. end of silking in corn). For planning purposes UNL recommends: 6 inches of irrigation water in eastern Nebraska; 9 inches in central Nebraska , 12 inches in western Nebraska; and 15 inches in the Panhandle.

Cols. c & d:  Planned crop and expected yield. Expected yield should be the average yield of the past five harvests of this crop on this field (excluding years with unusual stress), multiplied by 1.05. (from Inventory Form 4, Part 1 -  p. 12)  Growers are discouraged from applying manure to fields where the next crop will be grown for human consumption.

Col. e:  Soil test nitrate-N1 (in ppm) is the weighted average of the pre-plant soil test nitrate-N in the root zone - a minimum of 24" deep.  See Table R-2 for calculating weighted average soil nitrate level.

Col. f:  Enter the total nitrogen need for the crop to be grown using the recommendations in Reference Tables R-5 through R-17.  In most cases this includes credits for organic matter nitrogen release (if greater than 3.0 percent, use 3.0) and soil test nitrate.  Only if a nitrogen recommendation table for a crop is not available should you estimate nitrogen removal by multiplying the appropriate value from Reference Table R-1 by the yield.

Because legumes fix their own nitrogen, they may not utilize as much of the manure nitrogen as a grass type crop.  Therefore, a factor is given for legumes at the bottom of Reference Table R-1 to multiply by the nitrogen removal rate for an environmentally sound recommendation.

Col. g:  Enter the total of values from prior manure applications from Form 17, Columns k, l, and m.  (Leave this column blank the first year you use this form unless you have actual values to enter.)

Col. h:  Enter an irrigation credit by multiplying the nitrate-N concentration of the irrigation water by the acre-inches of water usually applied by mid season (e.g. end of silking in corn), and by the conversion factor  0.227.

Example: Irrig. credit (lbs. N/ac.)  =  9 inches applied  X  10 ppm Nitrate-N  X  0.227 = 20.

Col. i:  Enter a legume or green manure nitrogen credit from Reference Table R-4.

Col. j:  Enter fertilizer N applied since the last harvest (e.g. as 11-52-0), or planned in addition to manure nitrogen (e.g. starter N).

Col. k:  Net nitrogen need before applying manure. Column f  -  Columns g, h, i, and j.

Col. l:   Enter the line number from Form 17, Column a of the manure source, rate and incorporation schedule planned to be applied to this field.  If none is to be applied this year, enter zeros in Columns l, m, and n.

Col. m:  Enter the planned manure application rate that will meet most or all of the nitrogen needs in Column k.

Col. n:  Enter the nitrogen available at the manure application rate in 
Column m.  If this is significantly above the nitrogen need 
(Column. k), then recalculate the line in Form 17 reducing application rates or delaying incorporation if possible.

Col. o:  Extra fertilizer N needed to meet crop needs  (Column  k – Column  n). If net nitrogen is within +20 lbs., no adjustment is necessary.  A larger negative number suggests the need for a lower manure or commercial fertilizer application rate.  A positive number suggests the need for more manure or fertilizer.

#### Form 18 . Annual Field Plan for Nitrogen

Complete the line for the next year before applying any manure.

Field or Management Area:      		Soil Organic Matter:  Yr.:              ,          %.         Yr.:              ,         .%

If irrigated: Acre-inches / year (for Col. h):          	   NO3-N conc. of irrigation water:  Yr.:           ,        ppm.          Yr.:           ,       ppm.

| a. Year | b. Previous Crop | c. Planned Crop | d. Expect-ed Yield (E.Y.) bu./ac., Tons/ac., lb./ac. (Avg. x 1.05) | e. Soil Test Nitrate-N (average ppm) | f. Total Nitrogen Need or Removal (Ref. Table R-5 to R-17) (lbs./acre) | Nitrogen Credits (lbs./acre) |  |  | j. Fertilizer Nitrogen Credit (Starter, etc.) (lbs./ac.) | k. Net N Need Before Manure Applica- tion (Cols. f–g – h – i – j) (lbs./ac.) | l. Manure Applic. Option (write line no. from Form 17, Col. a) | m. Planned Manure Applica-tion Rate (Form 17, Col. c) (tons/ac, gal./ac, or in./ac.) | n. Rate of Manure Nitrogen Available (Form 17, Col. j) (lbs./ac.) | o. Extra Nitrogen Needed as Fertilizer (Cols. k-n) (lbs./ac.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | g. Manure N from Past Years (Form 17, Col. k-m) (lbs./ac.) | h. Irrigation Water N (ppm x 0.227 x Ac.-in.) (lbs./ac.) | i. Legume / Green Man. N (Ref. Table R-4) (lbs./ac.) |  |  |  |  |  |  |
| 2001 | Soybeans | Corn | 170 | 3 ppm | 167 at 2% OM | 0 | (10 ppm) 20 lb/ac | 45 | 6 | 96 | Ex | 18 T./ac. | 108 | -12 lb/ac |

51

Annual Plan

52

Good

Steward

Instructions for Annual

Field Plan for Phosphorus (Form 19)

Purpose

This optional planning guide will determine if phosphorus (including manure nitrogen) is being applied in approximate balance with crop phosphorus requirements or removal.  All phosphorus annual planning documents are organized to maintain multiple years’ records for a single field on one page.  Make additional copies of Form 19 so that one blank form is available for each field receiving manure. A Manure Use Plan spreadsheet, available at http://cnmp.unl/cnmpsoftware.html, completes these same calculations.

Regulations

For management of phosphorus, NDEQ’s Title 130 requires the producer:

- to define the frequency of soil sampling and analysis for available soil phosphorus levels (typically every three years),

- to conduct the sampling for areas not to exceed 40 acres prior to any manure application, and

- to report to NDEQ all samples where phosphorus exceeds 150 ppm Bray 1 or 100 ppm Olsen.

Form 19 is not mandatory; however, it will provide insight as to the balance of phosphorus for individual fields and identifies those fields that are at risk for exceeding the 150 ppm reporting threshold.

Instructions

Fill in the field or management area name at the top of the page.

Col. a:  One line will represent each crop year.

Col. b:	Identify manure handling systems supplying manure.

Col. c:  Enter the planned manure application rate and check the correct terms.

Col. d:  Enter the phosphorus concentration from manure analysis.  If manure analysis is not available, Reference Table R-3 may provide an approximate nutrient concentration.

Col. e:  Enter the phosphorus availability factor.  If the Bray 1 or Mehlich III phosphorus soil test is < 30 ppm, or the Olsen test is < 20 ppm (in col. i), assume phosphorus availability to the crop is 0.7 the first year (the rest – 0.3 – is available the next year).  If  soil test P levels are greater that these values, assume all the P (1.0) is available the first year.  Soil test level does not impact manure phosphorus availability.  Higher soil P levels provide a safety factor if all manure P does not become available for crop use.

Col. f:  Calculate the manure phosphorus credit (Cols. c x d x e) .

Crop phosphorus balance may be estimated as follows:

Cols. g & h:  Enter the planned crop and expected yield.  Expected yield should be estimated as the average of the past five years of this crop in this field multiplied by 1.05. (Exclude years with unusual stress.)

Col. i:	Enter this year’s soil test results in ppm and analysis method.

Col. j:	Look up the P (P2O5) recommendation for Column i in Reference Tables R-18 through R-32.  If a P recommendation table is not available for this crop, then go to Column l instead.

Col. k:	Enter any planned fertilizer P2O5 application.

Col. l: 	(Use Columns l and m only if there is no P recommendation table.) Enter the phosphorus removal rate from Reference Table R-1 in lbs. P2O5 per unit.

Col. m:	Multiply Column l by the Expected Yield to provide a rough estimate of crop P requirement.  However, a soil test recommendation is always the preferred method of estimating crop requirements.

Col. n: 	Construct a phosphorus (P2O5) balance for this field by subtracting Total Phosphorus Removed from the planned fertilizer and manure credits (Cols. f + k - m).

If the balance is negative, then additional commercial fertilizer or manure is required to meet the phosphorus needs of the crop this year.

If the balance is positive by more than 25 pounds and the P Index for this field is High or Very High, reduce the manure application rate.

Col. o: 	Calculate the potential soil test increase or decrease by dividing Column n by 20.  If  Column o is positive, the soil test will increase; if negative, the soil test will decrease.  Actual changes in soil phosphorus levels may not be accurately reflected by this calculation due to the complexities of soil chemistry for phosphorus.

Good

Steward

##### Form 19. Annual Field Plan for Phosphorus

Field or Management Area:

Soil phosphorus  tests above 150 ppm Bray 1 or Mehlich III, or 100 ppm Olsen tests must be reported to NDEQ.

| a. X Crop Year | Manure Phosphorus Availability |  |  |  |  |  |  | Crop Phosphorus Balance |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | b. Manure Handling System | c. Planned Manure Application Rate |  | d. Manure Phosphorus (P2O5) Concentration from Analysis X |  | e. Phos. Avail-ability Factor (0.7 or 1.0) | f. Phos- phorus Manure Credit (c x d x e) (lb./ac.) | Planned Crop |  | i. Soil Test Phos-phorus (ppm) & Method | j. Phos-phorus (P2O5) Recom-menda-tion (lb./ac.) | k. P Fer-tilizer Applica-tion (P2O5) (lbs./ac.) | Crop P (P2O5) Removal (use only if no soil test is available) |  | n. P2O5 Balance (Cols. f + k - m) (lbs./ac) | o. Potential Soil P2O5 Increase or Decrease (n ÷ 20) (ppm) |
|  |  |  |  |  |  |  |  | g. Name | h. Expect-ed Yield |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | l. Factor (See Table R-1) | m. Total P Removed (h x l) |  |  |
| 2001 | Beef, dirtlot | 20 | Ton/ac 1000 gal/ac ac-in/ac | 18 | lbs./ton lbs./1000 gal lbs./ac-in | 0.7 | 250 lbs./acre | Corn | 150 bu/ac. | 20 Bray-1 | Row 0 Bdcst 0 | 10 (pop-up) | 0.3 lbs./bu | 45 lbs./ac. | 215 lbs./ac. | 11 ppm |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  | lbs./ton lbs./1000 gal lbs./ac-in |  | lbs./acre |  |  |  | lbs./acre |  |  | lbs./acre | lbs./acre |  |

Annual Plan

53

Instructions for Action Plan (Form 20)

54

## Purpose

The results of the nitrogen and phosphorus management plans should be summarized into an Action Plan that is carried to the field for defining:

- Which fields are to receive manure from which storage system;

- How much manure and other nutrients are to be applied;

- When manure should be applied and incorporated within X days.

A Manure Use Plan spreadsheet, available at http://cnmp.unl.edu/cnmpsoftwarehtml, completes these same calculations.

Regulations

NDEQ’s Title 130 requires that a permit application have a nutrient management plan that includes… “waste sampling and analysis procedures, land application soil sampling and analysis procedures, and planned application rates, methods, and frequencies.”  To comply with these regulations, the producer will need to:

- Annually prepare an Action Plan and keep this plan on file for five years (for NDEQ inspection);

Instructions

- Suggested manure application rate, estimated for individual fields in Form 18 (nitrogen based), should be transferred to the Action Plan.  If soil P Index is High or Very High, manure application rates based on a phosphorus balance (Form 19) or no manure application would be recommended.  Assumptions relative to incorporation of manure were made in Form 17.  Recommendations for incorporation recorded in the Action Plan should be compatible with those assumptions.

- Suggested timing of manure application is based on risk of runoff, availability of cropland, labor availability and other factors.

For fields with high P Index levels, winter months would be undesirable for land application.  For fields susceptible to compaction, wet months (e.g. spring months) also might be unacceptable.

- Where manure and other nutrient credits do not meet crop nutrient requirements, additional commercial fertilizer may be needed.  Check Forms 18 and 19 for commercial fertilizer recommendations.

4.	For a desired application rate, identify the appropriate equipment operational settings (table at bottom of Form 20) that will produce the desired manure application rate.  If these settings are unknown, the equipment will need to be calibrated and those operational settings identified for future reference.  See Manure Application Calibration Guide, page 87, for options for calibrating equipment.

Form 20. Action Plan

This document should be photocopied and carried to the field during land application.

Crop Year:

| X X Field ID | Manure System | Planned Manure Application Rate |  | Incorporate Into Soil? | Manure Nutrient Applic. Rate (lbs./acre) |  | Suggested Timing of Manure Application | Commercial Fertilizer Rate (lbs./acre) |  | Application Instructions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | N | P2O5 |  | N | P2O5 |  |
| Sample X X North Pivot | Beef Finish, dirt lot | 18 | Ton/ac 1000 gal/ac ac-in/ac | Yes, No 1 days | 90 | 360 | J F M A M J J A S O N D | 0 | 0 | 30 ft. creek setback |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac | Yes, No days |  |  | J F M A M J J A S O N D |  |  |  |

| Application Rate | Tractor Settings |  | Applicator Settings |  | Pivot Settings |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gear | RPM | PTO / hydraulic | Chain sp. or orifice | % speed | psi / gpm | Dilution: ? manure /1 gal. water |
|  |  |  |  |  |  |  | gallons manure |
|  |  |  |  |  |  |  | gallons manure |

55

Annual Plan

56

Inventory

Strategic Plan

Annual

Plan

Review and

RecordsPlan Modification

Manure Application Field Records59

Nutrient Status Indicator63

Continuing Education Summary64

Manure Transfer to Off-Farm Users 65

Reports of High Soil Phosphorus Levels66

Livestock Waste Discharge Notification67

Review as Necessary

Review Annually

NDEQ Permit Application

NDEQ Permit Implementation

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Records

57

An electronic copy of this workbook and individual worksheets are available online at http://cnmp.unl.edu

Documentation and Records Forms (21 To 28)

58

Purpose

The purpose of this section is to provide sample forms for the record keeping requirements of permits for NDEQ’s Title 130 and EPA’s National Pollutant Discharge Elimination System (NPDES) (typically applied only to feedlots with over 1000 head of capacity).

NDEQ Title 130

As of February 28, 2000, Title 130 requires that a livestock producer operating under an NDEQ Livestock Waste Facility Control Permit must maintain appropriate records documenting implementation of an annual crop nutrient management plan and other facility maintenance activities.  These records must be maintained for a five-year period and available for inspection by NDEQ field inspectors.  Records of current and past annual plans for nitrogen management are required.

Instructions

It is recommended that in order to document a farm’s implementation of a “Nutrient Management and Land Application Plan,” the following records should be maintained for at least five years.  Additional records related to management of the manure storage or holding pond are detailed in the “Manure Storage and Open Lot Runoff Storage Workbook.”  Records with an asterisk are required by NDEQ.

- Planning documents and calculations for:

- NDEQ permit application document with all Inventory and Strategic Plan documents*

- Crop available manure nitrogen (Form 17)*

- Annual nitrogen management plan for each year (Form 18)*

- Annual phosphorus management plan for each year (Form 19)

- Action plan (Form 20)*

2) Records for:

- Manure analysis.  Summaries of key manure analysis results can be included in Form 30.*

- Soil tests.1  Summaries of key soil test information can be included in Form 31.*

- Manure application field records (Forms 21, 22, 23, or 24)*

- Additional crop, soil and water nutrient status indicators (Form 25)

- Land application site agreements (Form 14)*

- Continuing Education Summary (Form 26)

- Off-farm transfers of manure (Form 27)*

- Reports of soil phosphorus tests exceeding 150 ppm, Bray 1 or Mehlich, or 100 ppm, Olsen (Form 28)*

- Livestock Waste Discharge Notification reports (Form 29)*

- Crop yields (Form 31)*

3) Review of past year and future plan modifications for:

- Post cropping season summary of crop yields, manure analysis and application rates (Form 30 and 31)*

- Modifications to Next Year’s Plan

Form 21.  Solid Manure Application Field Record

Farm Owner:____________________________	Livestock/Poultry Facility:__________________________ Year:______________

Manure Spreader:________________________	Net Load Capacity:	__________________  tons

| Date/ Time | Field ID or Management Area | Number of Loads | Is Manure Incorporated into Soil? X 1 | Area Covered (acres) | Setbacks Maintained 1 | Precipitation and Other Weather Conditions: 24 hrs. 24 hrs Prior Today After |  |  | Soil/Field Conditions | Operator Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample | # Home 80 | / / / / / / / / / / / | ____Yes, _____days ____No later | 12 ac. | 30’ from Cow Creek | No rain | No rain SE wind | 0.25” rain S wind | ___Frozen ___ Snow-covered X ___ Wet ___ Moist ___ Dry | JMK |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes, _____days ____No later |  |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |

59

1.  State permitted operations: A 30-foot minimum setback is required between the edge of a stream channel, pond, or lake and the manure application. 100-foot setbacks are sometimes required. No setback is required from wetlands or grass waterways if they are not part of a stream or lake, but manure cannot be applied to standing or flowing water.  EPA’s NPDES permit will require setbacks of 100 feet or a permanent vegetated buffer of 35 feet from surface waters. Setbacks should be illustrated on an aerial map (Form 4, Part 2).

Records

60

Form 22.  Slurry or Sludge Application Field Record

Farm Owner:____________________________	Livestock/Poultry Facility:__________________________   Year:______________

Manure Applicator:________________________	Net Load Capacity:	_________________  gallons

| Date/ Time | Field ID | Number of Loads | Is Storage Agitated At Pump Out? X | Is Manure Incorporated into Soil? X | Area Covered (acres) | Setbacks Maintained 1 | Precipitation and Other Weather Conditions: 24 hrs. 24 hrs Prior Today After |  |  | Soil/Field Conditions | Oper-ator Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample | # Pivot 1 | / / / / / / / / / / / | ____Yes ____No | ____Yes, __1__days ____No later | 8 acre | 30’ from Cow Creek | No rain | No rain SE wind | 0.25” rain S wind | ___Frozen ___ Snow-covered X ___ Wet ___ Moist ___ Dry | JMK |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  |  | ____Yes ____No | ____Yes, _____days ____No later | acre |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |

4

1.  State permitted operations: A 30-foot minimum setback is required between the edge of a stream channel, pond, or lake and the manure application. 100-foot setbacks are sometimes required. No setback is required from wetlands or grass waterways if they are not part of a stream or lake, but manure cannot be applied to standing or flowing water.  EPA’s NPDES permit will require manure application setbacks of 100 feet or a permanent vegetated buffer of 35 feet from surface waters with no manure application.  Setbacks should be illustrated on an aerial map (Form 4, Part 2)

Form 23.  Towed Hose or Irrigation System Field Record of Manure Application

Farm Owner:____________________________	Livestock/Poultry Facility:__________________________ Year:______________

Manure Applicator:________________________	Manure Pumping Rate:	_________________  gpm

| Date/ Time | Field ID | Operating Hours Begin End 8:00 | Rate of Clean Water Addition | Is Storage Agitated At Pump Out? | Is Manure Incorporated into Soil? | Area Covered (acres) | Setbacks Maintained1 | Precipitation and Other Weather Conditions: 24 hrs. 24 hrs Prior Today After |  |  | Soil/Field Conditions | Oper-ator Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample 3/30/00 | # Pivot 1 | _______am ______ am 4:30 _______pm ______ pm | 2 to 1 | ____Yes X ___No | ____Yes, _____days X ____No later | 130 ac. | 30’ from Cow Creek | No rain | No rain SE wind | 0.25” rain S wind | ___Frozen ___ Snow-covered X ___ Wet ___ Moist ___ Dry | # RK |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | _______am ______ am _______pm ______ pm | to 1 | ____Yes ____No | ____Yes, _____days ____No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |

61

1.  State Permitted operations: A 30-foot minimum setback is required between the edge of a stream channel, pond, or lake and the manure application. 100-foot setbacks are sometimes required. No setback is required from wetlands or grass waterways if they are not part of a stream or lake, but manure cannot be applied to standing or flowing water.  EPA’s NPDES permit will require manure application setbacks of 100 feet or a permanent vegetated buffer of 35 feet from surface waters with no manure application.  Setbacks should be illustrated on an aerial map (Form 4, Part 2).

Records

62

Form 24.  Irrigation Field Record of Manure Application (If Application Rate is Known)

Farm Owner:____________________________	Livestock/Poultry Facility:__________________________ Year:______________

Manure Applicator:________________________	Manure Pumping Rate:	_________________  gpm  or   _______________ac-inches/hr

| Date/ Time | Field ID | Depth of Irrigation Application (inches) | Rate of Clean Water Addition | Is Storage Agitated At Pump Out? | Is Manure Incorporated into Soil? | Area Covered (acres) | Setbacks Maintained1 | Precipitation and Other Weather Conditions: 24 hrs. 24 hrs Prior Today After |  |  | Soil/Field Conditions | Oper- ator Initials |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample 3/30/00 | Pivot 1 0.75 | inch(es) | 2 to 1 | ____Yes ____No X | ___Yes, ____days X ___No later | 130 ac. |  | No rain | No rain SE wind | 0.25” rain S wind | ___Frozen ___ Snow-covered X ___ Wet ___ Moist ___ Dry | # RK |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |
|  |  | inch(es) | to 1 | ____Yes ____No | ___Yes, ____days ___No later | ac. |  |  |  |  | ___Frozen ___ Snow-covered ___ Wet ___ Moist ___ Dry |  |

1.  State Permitted operations: A 30-foot minimum setback is required between the edge of a stream channel, pond, or lake and the manure application. 100-foot setbacks are sometimes required. No setback is required from wetlands or grass waterways if they are not part of a stream or lake, but manure cannot be applied to standing or flowing water.  EPA’s NPDES permit will require manure application setbacks of 100 feet or a permanent vegetated buffer of 35 feet from surface waters with no manure application.  Setbacks should be illustrated on an aerial map (Form 4, Part 2)

Good

Steward

Form 25. Crop, Soil, and Water Nutrient Status Indicators

Instructions:  Record any relevant information below that may provide insight as to the nitrogen status of the crop or soil.

| Field ID or Management Area | Pre-Sidedress Soil Nitrate Test |  | Chlorophyll Meter Readings |  |  |  | Post-Season Stalk Tissue |  | Other Observations or Field Test |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date | Content (ppm) | Date | Growth Stage | Reading | Reading - % of Reference | Date | Nitrate Conc. (ppm) | Date | Observation |
| Example | 6/10 | 15 | 7/15 | V 18 | 45 | 98 | 10/1 | 1500 | 8/15 | Lower 3 leaves slightly yellow |

Instructions:  Summarize any available water quality measurements.

| Field ID | Irrigation Well1 |  | Nearby Well1:__________ |  | Surface Water Measurements |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Date | Nitrate (ppm) | Date | Nitrate (ppm) | Date | Observation |
| Sample | 7/5 | 14 |  |  |  |  |

63

1. Nitrate, ammonium, or coliform bacteria can be measured to provide an indication of contamination by fertilizer and/or manure.

Records

64

Form 26.  Continuing Education Summary

| Date | Educational Program and Location | Time Involved | Who Taught Program? | Who Organized Program? | Who Attended? |
| --- | --- | --- | --- | --- | --- |
| Example: 3/1/00 | Nutrient Mgmt. Planning /Kearney | 4 hrs. | ### UNL Cooperative Ext. | NE Cattlemen | John Doe |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |
|  |  | hrs. |  |  |  |

NDEQ Title 130 requires the owner’s attendance at an approved land application educational program every five years.

Form 27.  Manure Transfer to Off-Farm Users

Method of verification of manure transfer amounts:

___ Scale	___ Flow meter	___ Count of loads	___ Other:________________________________________________

| Date X | Off-Farm User Name/Address | Employee Making Entry | Amount of Transfer | Manure Analysis |  | Total Nutrient Transfer |  | Location of Field Receiving Manure |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | N | P2O5 X | N (lbs) | P2O5 (lbs) |  |
| Mar. 6-9, 2001 16 19 2,000 | John Corn Grower, RR 2, Anytown NE | Jim Part Time | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in | 32,000 | 38,000 | Corn Grower’s Home 80 (1 mi north of feedlot) |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
|  |  |  | Tons Gals. Ac-In. |  | Lbs./ton Lbs./1000 gal Lbs./ac-in |  |  |  |
| TOTAL |  |  | Tons Gals. Ac-In. |  |  |  |  |  |

65

Records

66

Form 28.  Report of High Soil Phosphorus Tests

Regulations

Title 130 requires reporting of all soil samples over 150 ppm P (Bray 1 or Mehlich III) or 100 ppm P (Olsen) to NDEQ.

| Mail report to: |  | Producer name: |  |  |
| --- | --- | --- | --- | --- |
|  | NDEQ | Address: |  |  |
|  | Agriculture Section |  |  |  |
|  | 1200 N Street |  |  |  |
|  | P.O. Box 98922 | Phone number: |  |  |
|  | Lincoln, NE 68509-8922 | Date: |  |  |

| Field ID or Management Area 1 X | Location | Date of Test | Sample Depth | Soil Phosphorus Level (ppm) | Test Method |  |
| --- | --- | --- | --- | --- | --- | --- |
| Example | #1…North of Feedlot | 4/2001 | 0 - 6” | 205 | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |
|  |  |  |  |  | Bray Mehlich Olsen Other | Mehlich III |

1. Attach aerial map (preferably copy of one used in permit application) and identify fields being reported.

Instructions for Livestock Waste Discharge

67

Notification (Form 29)

Purpose

The following form is recommended by NDEQ for use in reporting any discharge, either planned or accidental, from a livestock waste control facility.

NDEQ Title 130

Title 130 requires that “Any discharge of waste shall be reported to the Department within 24 hours of the event and in a written report to the Department within seven days of the event. For any discharge, the Department may request the operation to supply rainfall, land application and system storage records for up to the previous 12-month period prior to the discharge event.”

A discharge of livestock waste from a Livestock Waste Control Facility (LWCF) is prohibited unless:

1. 	Such discharge is to prevent a facility failure, which would result in loss of life, personal injury or severe property damage; and

2. 	No feasible alternative exists; and

3. 	The permittee submits a notice to the Director of the Nebraska Department of Environmental Quality (NDEQ) as follows:

Records

Within 24 hours of becoming aware of a discharge or the need to discharge [contact 
NDEQ at (402) 471-4239 or for operations located in the western half of Nebraska, 
contact the NDEQ North Platte Office at (308) 535-8142]; and

- Within 7 days of a discharge, submit a written report; and

4. 	The discharge is conducted under such conditions as to minimize any adverse effects.

Instructions

Step 1.  If a discharge occurs, initiate the steps identified in your Emergency Response Plan.

Step 2.  Within 24 hours, notify NDEQ of the discharge.

Step 3.  Complete the following form and deliver to NDEQ at the address listed below within seven days of the discharge.

Step 4.  If you observe dead fish that could have resulted from the discharge, contact the 
Nebraska Game and Parks Commission immediately at (402) 471-0641. After hours, call (402) 471-4545.

Additional Information

Questions? Contact: Nebraska Department of Environmental Quality, Agriculture Section, P.O. Box 98922, Lincoln, NE 68509-8922; phone (402) 471-4239. Visit the NDEQ web site at http://www.deq.state.us.  The attached form and most of the information on this page are from NDEQ Environmental Guidance Document, Livestock Waste Discharge Notification, dated September 2001.

68

Form 29:  Livestock Waste

Discharge Notification

Adapted from NDEQ fact sheet “Livestock Waste Discharge Notification,” September 2001.

NAME:

Permitted Operation Name

OWNER/MANAGER:

ADDRESS:

P.O. Box, Street Address

City, State and Zip Code

Legal Description of Operation

,       	, of      	,      	 N,      	  E or  W,      	County

1/4          1/4               Section            Township 	    Range

Do you have an NPDES Permit?   Yes     No            If yes, Permit No:

Complete the following and submit a map or drawing of the operation, LWCF, discharge flow pattern and stream:

1 	List reason(s) for discharge (i.e., power failure, large storm or chronic wet period, leak or break in water supply system, component failure of the waste control facility; and/or releases during land application due to equipment failure, accidents or irrigation equipment failure):

2.	The discharge flowed into       	 and

(name of ditch, drainage way, stream)

into

(name of primary stream)

3. 	Did the discharge flow directly into surface water or did the discharge flow over cropland prior to discharging to surface water?

4. 	The approximate width and depth of the surface water (which the discharge entered):

(width in feet) and      	 (depth in feet)

5. 	Date and time the discharge started and ended.  Please indicate whether the start time was the actual time or was when the discharge was discovered.

6. 	The average flow of the discharge was :      	 (gallons/minute)

7. 	Estimated total volume of discharge (cu.-ft.):      	(L x W x D)

8. 	Describe any damage to the LWCF:

69

9. 	Describe actions taken, and factors and conditions that helped minimize any adverse effects to the environment from the discharge:

10. 	Describe any obvious or known impacts to the environment from the discharge:

OPTIONAL INFORMATION

1. 	On a case-by-case basis, the Nebraska Department of Environmental Quality may require sampling.  If not required by the Department, the operation may want to provide the Department with documentation that the discharge did not impact waters of the State or the discharge was conducted in a manner to reduce adverse effects to the environment. The following sampling procedure has been outlined:

Please include procedures taken toward quality control on handling the samples.  Include information on the time when the samples were collected and when the lab received the samples.  You may wish to contact the lab for special sampling and handling instructions for the samples in order to eliminate contamination of the samples.

	Was sample kept cool (with ice) in the delivery/holding time?   Yes     No

	The following items should be included in the analysis. Sample locations, at a minimum, must include point of discharge, 100 feet upstream of the discharge, 100 feet downstream of the discharge and the mixing zone (where the discharge mixes with surface water). Provide a map with collection sites marked.

a)	5-day Biochemical Oxygen Demand (BOD-5);

b) 	total ammonia-nitrogen as nitrogen;

- nitrate-nitrite;

- Total keldahl nitrate;

e) 	pH;

f) 	temperature of the effluent and receiving stream;

g) 	sodium;

h) 	total phosphorus;

i) 	chlorides;

j) 	Chemical Oxygen Demand (COD);

k) 	Fecal Coliform Bacteria

I HEREBY CERTIFY THAT THE INFORMATION SUBMITTED HEREIN IS TRUE AND CORRECT TO THE BEST OF MY KNOWLEDGE AND BELIEF.

X ______________________________________________________	________________________

Signature of Authorized Representative				Date

If you observe dead fish that could have resulted from the discharge, contact the Nebraska Game and Parks Commission immediately at (402) 471-0641.  After hours, call (402) 471-4545.

Questions?  Contact Nebraska Department of Environmental Quality, Agriculture Section, P.O. Box 98922, Lincoln, NE  68509-8922; phone (402) 471-4239.  Visit their Web site http://deq.state.ne.us

70

Inventory

Strategic Plan

Annual

Plan

Review and

Plan Modification

Manure Analysis and

Application Rate73

Summary of Crop Yield74

Records

Review as Necessary

Review Annually

Nebraska’s

C N M P

Comprehensive Nutrient Management Plan

Review and Plan Modification

NDEQ Permit Application

NDEQ Permit Implementation

71

An electronic copy of this workbook and individual worksheets are available online at http://cnmp.unl.edu

72

Instructions for Post Cropping Season

Summary (Form 30 & 31)

Purpose

1.	Summarize actual manure application rate and manure analysis based on field records and equipment calibration records.

- Modify future years’ nitrogen credit and modify future recommended application rates.

NDEQ Title 130

As of June 18, 2001, Title 130 requires that a livestock producer operating under an NDEQ Livestock Waste Facility Control Permit must maintain appropriate records documenting implementation of an annual crop nutrient management plan and other facility maintenance activities.  These records must be maintained for five years and be available for inspection by NDEQ field inspectors. NDEQ requires a record of locations and quantities of livestock manure land applied, nutrient value of livestock manures, and records of waste sold or given away.

Sample Forms Provided

- Form 30.  Post cropping season summary:  Manure analysis and application rate

- Form 31.  Post cropping season summary:  Crop yield

Instructions

Form 30

- Enter planned manure application rate from Action Plan (Form 20) and actual application amounts from field records (Forms 21 – 24).

- Enter planned manure nutrient concentrations from Form 17 (nitrogen) and Form 19 (phosphorus) and actual manure nutrient concentration from manure sample report.

- Enter moisture content from manure sample report.

- Is actual nitrogen and phosphorus application rate (manure application rate X nutrient concentration) within 25 percent of planned application rates.  If a discrepancy exists, is it due to:

- application rate? Consider recalibrating manure application equipment or adjusting application rate used in planning to reflect the most recent calibration check.

- manure nutrient concentration and manure moisture content? Variations in manure moisture are the most common cause for changes in manure nutrient concentration.  Is it possible to get a representative manure sample just prior to application so that application rates can be adjusted? Can you predict when manure may be drier (higher nutrient concentration) or wetter (lower nutrient concentration) and make last minute adjustments to planned application rates?

- Was the check of nitrogen made in time to adjust any commercial nitrogen fertilizer application?

- A manure nitrogen application rate significantly different from the planned application rate may require an adjustment in the estimated manure nitrogen availability for next year’s crop.  Review your calculations of organic-N availability for next year completed in Form 17 for changes from the original plan.

Form 31

1.	Update record of crops grown and actual yields for individual fields using Form 31 or comparable records.

Form 30.  Post Cropping Season Summary:

Manure Analysis and Application Rate

Year:

| Field ID or Management Area | a. Manure Application Rate |  |  | Manure Moisture Content | b. Organic-N Content of Manure |  | Organic N Application Rate1 (a x b) |  | c. Ammonium-N Content of Manure |  | Ammonium –N Application Rate1 (a x c) |  | d. P205 Content of Manure |  | P205 Application Rate (a x d) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Planned | Actual X |  | (%) | Planned | Actual | Planned | Actual | Planned | Actual | Planned | Actual | Planned | Actual | Planned | Actual |
| Sample | 25 | 29 | Ton/ac 1000 gal/ac ac-in/ac | 75% | 12 | 17 | 25 x 12 x 0.25 = 75 | 29 x 17 x 0.25 = 123 | 2 | 4 | 25 x 2 x 0= 0 | 29 x 4 x 0= 0 | 18 | 21 | 25 x 18 = 450 | 29 x 21= 609 |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review |  |  | Ton/ac 1000 gal/ac ac-in/ac |  |  |  |  |  |  |  |  |  |  |  |  |  |

73

1.  To improve the accuracy of the ammonium and organic nitrogen comparison of planned and actual application rates, you may want to include the appropriate crop availability factors.  Those factors can be located in Figure 1, page 48.

74

Form 31.  Post Cropping Season Summary of Crop Yields

Field or Management Area:

| Crop Year | Cropping Information |  |  | Soils Analysis Information |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Crop | Yield | Source1 | % OM | Soil P | Soil Nitrate and Sample Depth |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

Field or Management Area:

| Crop Year | Cropping Information |  |  | Soils Analysis Information |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Crop | Yield | Source1 | % OM | Soil P | Soil Nitrate and Sample Depth |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

Field or Management Area:

| Crop Year | Cropping Information |  |  | Soils Analysis Information |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Crop | Yield | Source1 | % OM | Soil P | Soil Nitrate and Sample Depth |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

1.  The source for crop yields.  Certified crop yields such as FSA records may be required by NDEQ.

75

CNMP Reference Tables

Reference Tables in the Nebraska CNMP Workbook

Table	  Topic									Page

R-1	Plant nutrient uptake	76

R-2	Calculation of weighted soil nitrate-N credit	77

R-3	Typical crop available nutrient content of manure	77

R-4	Legume and green manure nitrogen credits	77

UNL Soil Fertility Recommendations

|  | Nitrogen | Phosphorus |
| --- | --- | --- |
| Crop | Table and Page | Table and Page |
| Alfalfa | - | R-18, page 83 |
| Barley | R-5, page 78 | R-19, page 83 |
| Corn and corn silage | R-6, page 78 | R-20, page 83 |
| Dry beans | R-7, page 79 | R-21, page 84 |
| Grass | R-8, page 79 | R-22, page 84 |
| Millet | R-9, page 80 | R-23, page 84 |
| Oats | R-10, page 80 | R-24, page 84 |
| Popcorn | R-11, page 80 | R-25, page 85 |
| Potato | R-12, page 80 | R-26, page 85 |
| Sorghum | R-13, page 81 | R-27, page 85 |
| Soybean | - | R-28, page 85 |
| Sugar beets | R-14, page 81 | R-29, page 85 |
| Sunflower | R-15, page 82 | R-30, page 86 |
| Wheat, spring | R-16, page 82 | R-31, page 86 |
| Wheat, winter | R-17, page 82 | R-32, page 86 |

R-33	Applicator Calibration Guide

University of Nebraska Cooperative Extension publications applicable 
to the Manure Application Workbook:

1.	Nutrient Management for Agronomic Crops in Nebraska (EC01-155)

2.	Fertilizer Suggestions for Corn (NebGuide G74-174)

3.	Fertilizer Suggestions for Soybeans (NebGuide G87-859)

4.	Determining Crop Available Nutrients from Manure (NebGuide G97-1335)

5.	Manure Applicator Calibration (NebGuide G95-1267)

6.	Sampling Manures for Nutrient Analysis (NebGuide G02-1450)

7.	Manure Testing: What to Request? (NebFact NF02-507)

8.	Using a Chlorophyll Meter to Improve N Management (NebGuide G93-1171)

9.	The Corn Stalk Nitrate Test (NebFact NF01-491)

10.	Manure Application Calibration Guide (EC03-182)

11.	Calculating the Value of Manure for Crop Production (NebGuide G03-1519)

765

Table R-1.  Plant nutrient uptake and removal in the harvested part of the crop.  Values are pounds per unit at a common sales and winter storage moisture (100% DM).

| Crop | Test Wt. | DM % | N | P2O5 | Units |  | Crop |  | DM % | N | P2O5 | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grain Crops |  |  |  |  |  |  | Forage Crops (taken as hay) 1, 2 |  |  |  |  |  |
| Barley (Grain) | 48 | 86 | 0.87 | 0.33 | lbs./bu. |  | Alfalfa, mid-bloom |  | 85 | 46.2 | 9.3 | lbs./ton |
| (Straw) |  | 90 | 12.7 | 2.9 | lbs./ton |  | Birdsfoot trefoil |  | 85 | 43.2 | 9.0 | lbs./ton |
| Buckwheat (Grain) | 48 | 85 | 0.80 | 0.34 | lbs./bu. |  | Bluestem, mature |  | 85 | 7.9 | 2.7 | lbs./ton |
| Corn (Grain) | 56 | 84.5 | 0.70 | 0.31 | lbs./bu. |  | Bluestem, early heading |  | 85 | 21.8 | 5.8 | lbs./ton |
| (Stover) |  | 85 | 17.7 | 3.5 | lbs./ton |  | Bromegrass, smooth, mid-bloom |  | 85 | 39.2 | 10.9 | lbs./ton |
| Millet, proso (Grain) |  | 90 | 1.9 | 0.64 | lbs./cwt. |  | Clover, red |  | 85 | 40.8 | 9.3 | lbs./ton |
| Oats (Grain) | 32 | 86 | 0.60 | 0.23 | lbs./bu. |  | Fescue, Tall, full-bloom |  | 85 | 35.1 | 12.5 | lbs./ton |
| (Straw) |  | 90 | 12.7 | 2.5 | lbs./ton |  | Millet, foxtail |  | 85 | 23.4 | 7.4 | lbs./ton |
| Rye (Grain) | 56 | 87 | 1.1 | 0.40 | lbs./bu. |  | Orchardgrass, late-bloom |  | 85 | 22.8 | 11.7 | lbs./ton |
| (Straw) |  | 90 | 8.6 | 3.7 | lbs./ton |  | Prairie hay, mature |  | 85 | 14.4 | 5.5 | lbs./ton |
| Sorghum (Grain) | 56 | 87 | 0.90 | 0.40 | lbs./bu. |  | Reed canarygrass |  | 85 | 28.0 | 9.3 | lbs./ton |
| (Stover) |  | 80 | 13.6 | 4.0 | lbs./ton |  | Small grain, boot |  | 85 | 34.0 | 11.7 | lbs./ton |
| Wheat (Grain) | 60 | 86.5 | 1.2 | 0.50 | lbs./bu. |  | Small grain, dough |  | 85 | 21.8 | 10.9 | lbs./ton |
| (Straw) |  | 90 | 10.1 | 2.1 | lbs./ton |  | Soybean hay |  | 85 | 45.7 | 12.8 | lbs./ton |
|  |  |  |  |  |  |  | Switchgrass |  | 85 | 21.8 | 5.8 | lbs./ton |
| Oil Crops |  |  |  |  |  |  | Timothy, mid-bloom |  | 85 | 26.4 | 9.0 | lbs./ton |
| Soybeans (Grain) | 60 | 87 | 3.5 | 0.79 | lbs./bu. |  | Vetch, hairy |  | 85 | 56.6 | 13.2 | lbs./ton |
| (Stover) |  | 90 | 15.8 | 2.5 | lbs./ton |  | Wheatgrass, western, early-bloom |  | 85 | 19.9 | 5.8 | lbs./ton |
| Sunflower, oil (Grain) | 25 | 90 | 29.1 | 13.0 | lbs./1000 lbs. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Sugar Crops |  |  |  |  |  |
| Silage Crops |  |  |  |  |  |  | Sugar beet roots |  | 20 | 3.5 | 1.6 | lbs./ton |
| Alfalfa, mid-bloom |  | 40 | 21.8 | 4.9 | lbs./ton |  | Sugar beet tops |  | 18 | 5.5 | 1.3 | lbs./ton |
| Corn silage |  | 35 | 9.0 | 3.2 | lbs./ton |  |  |  |  |  |  | of beets |
| Small grain, dough |  | 35 | 9.0 | 4.5 | lbs./ton |  | Food Crops |  |  |  |  |  |
| Sorghum |  | 30 | 9.0 | 3.0 | lbs./ton |  | Dry Beans |  | 90 | 4.1 | 1.1 | lbs./cwt. |
| Sorghum-sudan |  | 30 | 10.4 | 2.9 | lbs./ton |  | Popcorn (Grain) |  | 86 | 1.8 | 0.6 | lbs./cwt. |
|  |  |  |  |  |  |  | (Stover) |  | 85 | 17.7 | 3.5 | lbs./ton |
|  |  |  |  |  |  |  | Potatoes 3 |  | 22 | 7.0 | 2.5 | lbs./ton |
| 1When grazing forages, phosphorus removal is limited to that incorporated into the body of the calf, or the milk of a dairy cow. A 500 lb. weaned calf contains about 4 lb. of phosphorus (9 lb. P2O5), and backgrounding will add another 0.8 lb. of P |  |  |  |  |  |  |  |  |  |  |  |  |
| (1.8 lb. P2O5) per 100 lb of gain. Supplementation of cow and/or calf will partly replace the P removed. (e.g. 0.30% P x 2 lb./day x 60 days = 0.36 lb. P = 0.8 lb. P2O5) |  |  |  |  |  |  |  |  |  |  |  |  |
| 2Calves will retain about 12% of forage nitrogen (about 2.7 lb. N / 100 lb. live wt.). More N is volatilized from the urine and manure (15% of this N). On average figure about 75% of grazed forage N is retained in the field. |  |  |  |  |  |  |  |  |  |  |  |  |
| Also, creep feed at 15% C.P. x 2 lb./day x 60 days = 2.9 lb. N. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3Due to potato disease micro-organisms entering a field in manure, manure applications are |  |  |  |  |  |  |  |  |  |  |  |  |
| discouraged where potatoes are or may be in the rotation. |  |  |  |  |  |  |  |  |  |  |  |  |

Table R-2.  Calculation of weighted soil nitrate-N credit.

| 77 | Example |  |  | Your Farm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a. Nitrate Nitrogen | b. Sample Depth | a x b | a. Nitrate Nitrogen | b. Sample Depth | a x b |
| Nitrate Sample 1 | 11 | 0 to 8 in. | 11 X (8 – 0) = 88 |  |  |  |
| Nitrate Sample 2 | 7 | 8 to 24 in. | 7 X (24 – 8) = 112 |  |  |  |
| Nitrate Sample 3 | 4 | 24-36 in. | 4 X (36 – 24) =48 |  |  |  |
| Weighted Nitrate = (sum of a x b)  total sample depth |  |  | (88 + 112 + 48)  36 = 7 |  |  |  |

If the soil test report for nitrate-N is in lbs./ac, then divide that value by 0.3 and by the sampling depth in inches to convert to ppm.  Use a default value of 3.0 ppm nitrate-N for samples less than 24 inches deep (unless for shallow-rooted crops), or where no samples were taken.  If the effective root zone is less than 24 inches deep, prorate the nitrate-N credit on the basis of a full rooting depth, e.g. if the root depth is 18 inches, then divide ppm by 2 (half of 36 inches).

Table R-3.  Typical crop available nutrient content of manure.  These values may be used when a manure analysis is not available.

| Species | NH4 - N | Org. – N | P2O5 | K2O |  | Species | NH4 - N | Org. – N | P2O5 | K2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solid Manure (lbs. per ton)* |  |  |  |  |  | Sludge from anaerobic lagoon (lbs. per 1,000 gal.) |  |  |  |  |
| Beef (dirt lot) | 4 | 7 | 7 | 11 |  | Dairy | 4 | 17 | 20 | 16 |
| Beef (paved lot) | 5 | 9 | 9 | 13 |  | Swine | 6 | 19 | 52 | 76 |
| Swine | 6 | 10 | 9 | 95 |  | Beef (settling |  |  |  |  |
| Dairy | 2 | 8 | 3 | 6 |  | basin) | 10 | 42 | 40 | 17 |
| Broiler litter | 12 | 34 | 53 | 36 |  | Top water from lagoon or holding pond (lbs / ac-in)2 |  |  |  |  |
| Turkey litter | 8 | 32 | 50 | 30 |  | Beef | 41 | 4 | 10 | 203 |
| Layer | 12 | 22 | 51 | 36 |  | Swine | 50 | 29 | 17 | 86 |
| Slurry Manure (lbs per 1,000 gallons)1 |  |  |  |  |  | Dairy | 27 | 18 | 13 | 113 |
| Dairy | 6 | 25 | 15 | 19 |  |  |  |  |  |  |
| Beef | 8 | 21 | 18 | 26 |  |  |  |  |  |  |
| Swine (earthen pit) | 24 | 8 | 22 | 20 |  |  |  |  |  |  |
| Swine (deep pit) | 33 | 17 | 42 | 30 |  | 1From “Manure Characteristics,” MWPS-18-1. |  |  |  |  |
| Layer | 37 | 20 | 52 | 33 |  | 2To obtain pounds per 1,000 gallons, divide by 27. |  |  |  |  |

Table R-4.  Legume and green manure nitrogen credits.

| Previous Crop | Nitrogen Fertilizer Credits (lbs./acre) |  |
| --- | --- | --- |
|  | Medium/Fine Soils | Sandy Soils |
| Soybeans | 45 | 45 |
| Soybeans < 30 bu./ac. due to season-long stress | 1.0 lb./bu. | 1.0 lb./bu. |
| References Sugar beet tops, followed by dry beans | 100 | 100 |
| Alfalfa (70-100% stand, >4 plants/ft2) | 150 | 100 |
| Alfalfa (30-69% stand, 1.5 to 4 plants/ft2) | 120 | 70 |
| Alfalfa (0-29% stand, <1.5 plants/ft2) | 90 | 40 |
| Red or Sweet Clover (70-100% stand, >4 plants/ft2) | 120 | 80 |
| Red or Sweet Clover (30-69% stand, 1.5 to 4 plants/ft2) | 100 | 60 |
| Red or Sweet Clover (0-29% stand, <1.5 plants/ft2) | 70 | 30 |

Table R-5.  Nitrogen recommendations for barley.

78

| Expected | Soil Organic Matter (%) |  |  |
| --- | --- | --- | --- |
| Yield | 1 | 2 | 3 |
| (bushels per acre) | Pounds of Nitrogen to Apply Per Acre |  |  |
| 40 | 50 | 30 | 10 |
| 60 | 80 | 60 | 40 |
| 80 | 110 | 90 | 70 |

Table R-6.  Nitrogen recommendations for corn based on expected yield with adjustments for soil nitrate-nitrogen and soil organic matter.

|  |  | Corn Expected Yield (Bu/Acre) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60 | 80 | 100 | 120 | 140 | 160 | 180 | 200 | 220 | 240 |
|  |  | Corn Silage Expected Yield (Tons/Acre) |  |  |  |  |  |  |  |  |  |
| Residual Soil Nitrate Level |  | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 35 | 38 |
| ppm | Relative Level | Pounds of Nitrogen to Apply Per Acre |  |  |  |  |  |  |  |  |  |
| 3% soil organic matter |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Low | 60 | 75 | 90 | 105 | 120 | 135 | 150 | 165 | 185 | 200 |
| 6 | Low | 35 | 50 | 65 | 80 | 95 | 110 | 125 | 145 | 160 | 175 |
| 9 | Medium | 0 | 25 | 40 | 55 | 70 | 90 | 105 | 120 | 135 | 150 |
| 12 | Medium |  | 0 | 15 | 35 | 50 | 65 | 80 | 95 | 110 | 125 |
| 15 | High |  |  | 0 | 0 | 25 | 40 | 55 | 70 | 85 | 100 |
| 18 | High |  |  |  |  | 0 | 15 | 30 | 45 | 65 | 80 |
| 21 | High |  |  |  |  |  | 0 | 0 | 25 | 40 | 55 |
| 24 | Very high |  |  |  |  |  |  |  | 0 | 15 | 30 |
| 27 | Very high |  |  |  |  |  |  |  |  | 0 | 0 |
| 2% soil organic matter |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Low | 65 | 85 | 105 | 120 | 140 | 160 | 175 | 195 | 215 | 230 |
| 6 | Low | 40 | 60 | 80 | 95 | 115 | 135 | 155 | 170 | 190 | 210 |
| 9 | Medium | 20 | 35 | 55 | 75 | 90 | 110 | 130 | 145 | 165 | 185 |
| 12 | Medium | 0 | 15 | 30 | 50 | 70 | 85 | 105 | 125 | 140 | 160 |
| 15 | High |  | 0 | 0 | 25 | 45 | 60 | 80 | 100 | 115 | 135 |
| 18 | High |  |  |  | 0 | 20 | 40 | 55 | 75 | 95 | 110 |
| 21 | High |  |  |  |  | 0 | 15 | 35 | 50 | 70 | 90 |
| 24 | Very high |  |  |  |  |  | 0 | 0 | 25 | 45 | 65 |
| 27 | Very high |  |  |  |  |  |  |  | 0 | 20 | 40 |
| 1% soil organic matter |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Low | 75 | 95 | 115 | 140 | 160 | 180 | 200 | 225 | 245 | 265 |
| 6 | Low | 50 | 70 | 95 | 115 | 135 | 155 | 180 | 200 | 220 | 240 |
| 9 | Medium | 25 | 50 | 70 | 90 | 110 | 135 | 155 | 175 | 195 | 215 |
| 12 | Medium | 0 | 25 | 45 | 65 | 85 | 110 | 130 | 150 | 170 | 195 |
| 15 | High |  | 0 | 20 | 40 | 65 | 85 | 105 | 125 | 150 | 170 |
| 18 | High |  |  | 0 | 20 | 40 | 60 | 80 | 105 | 125 | 145 |
| 21 | High |  |  |  | 0 | 15 | 35 | 60 | 80 | 100 | 120 |
| 24 | Very high |  |  |  |  | 0 | 15 | 35 | 55 | 75 | 95 |
| 27 | Very high |  |  |  |  |  | 0 | 0 | 30 | 50 | 75 |
| 33 | Very high |  |  |  |  |  |  |  |  | 0 | 25 |
| 36 | Very high |  |  |  |  |  |  |  |  |  | 0 |
| Without a soil test for nitrate-N, assume 3 ppm; without a soil test for organic matter, assume 2%. |  |  |  |  |  |  |  |  |  |  |  |

Table R-7.  Nitrogen recommendations for dry edible beans. If the previous crop was alfalfa or sugar beets and the sugar beet tops were left in the field, no fertilizer nitrogen should be applied. Ample nitrogen will be present from alfalfa or sugar beet residues to supply the dry bean crop needs.

79

| Residual Soil Nitrate-N (30 inch sample) | Nitrogen to apply |
| --- | --- |
| (ppm) | (Pounds per acre) |
| <5.6 | 75 |
| 5.6 to 8.2 | 50 |
| 8.3 to 11.0 | 25 |
| > 11.0 | 0 |

Table R-8.  Nitrogen recommendations for Nebraska pastures and haylands

|  | Cool Season Grasses |  | Warm Season Grasses |  |
| --- | --- | --- | --- | --- |
| Zone | Pasture | Hayland | Pasture | Hayland |
|  | Nitrogen to Apply*( pounds per acre) |  |  |  |
| I | 80-120 | 100-150 | 60-90 | 75-100 |
| II | 50-80 | 60-90 | 40-75 | 50-80 |
| III | 40-60 | 50-75 | 25-50 | 40-60 |
| IV | 20-40 | 30-60 | 20-40 | 30-50 |
| * Use the higher rate when a full profile of subsoil moisture is present. |  |  |  |  |

Table R-8.  (continued)  Recommended nitrogen application rates for Nebraska irrigated pastures based on residual soil nitrate-N.

|  | Nitrogen to Apply (pounds per acre) |  |  |
| --- | --- | --- | --- |
| Stocking Rate | 0 – 50 | 50 – 100 | 100 – 150 |
| (yearlings per acre) | For Soil Residual Nitrate (ppm in soil to 6 ft depth) |  |  |
| 3 | 8.3 | 5.6 | 3.7 |
| 4 | 11.1 | 8.3 | 6.5 |
| >4 | 12.5 | 11.1 | 9.3 |

References

Table R-9.  Nitrate-nitrogen indices and nitrogen recommendations for a three-foot soil sampling depth for millet production.

80

|  |  | Nitrogen to Apply |  |
| --- | --- | --- | --- |
| Residual Soil Nitrate Level |  | Following Fallow | Following Wheat |
| ppm | Relative level | Pounds per acre |  |
| 0 to 2.0 | Very low | 40 | 80 |
| 2.1 to 3.5 | Low | 20 | 60 |
| 3.6 to 5.0 | Medium | 10 | 50 |
| 5.1 to 8.0 | High | 0 | 30 |
| > 8.0 | Very high | 0 | 0 |
| * Average nitrate-N in a three-foot profile |  |  |  |

Table R-10.  Nitrogen recommendations for oats.

| Expected Yield – Oats | Soil Organic Matter (%) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |
| Bushels per acre | Pounds of N to apply per acre |  |  |  |
| 60 | 70 | 50 | 30 |  |
| 80 | 90 | 70 | 50 |  |
| 100 | 110 | 90 | 70 |  |

Table R-11.  Nitrogen recommendations for popcorn.

| Residual Soil Nitrate Level | Expected Yield |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hundred weight per acre |  |  |  |  |  |  |  |  |
|  | 25 | 30 | 35 | 40 | 45 | 50 | 55 | 60 | 70 |
| (ppm) | Nitrogen to Apply (pounds per acre) |  |  |  |  |  |  |  |  |
| 3% soil organic matter |  |  |  |  |  |  |  |  |  |
| 2 | 60 | 70 | 75 | 85 | 95 | 100 | 110 | 120 | 135 |
| 4 | 45 | 55 | 60 | 70 | 75 | 85 | 95 | 100 | 120 |
| 6 | 30 | 35 | 45 | 55 | 60 | 70 | 80 | 85 | 105 |
| 8 | 10 | 20 | 30 | 35 | 45 | 55 | 60 | 70 | 85 |
| 10 |  | 5 | 15 | 20 | 30 | 40 | 45 | 55 | 70 |
| 15 |  |  |  |  |  |  | 5 | 15 | 30 |
| 20 |  |  |  |  |  |  |  |  |  |
| 2% soil organic matter |  |  |  |  |  |  |  |  |  |
| 2 | 65 | 75 | 85 | 95 | 105 | 115 | 125 | 135 | 150 |
| 4 | 50 | 60 | 70 | 80 | 90 | 100 | 105 | 115 | 135 |
| 6 | 35 | 45 | 55 | 65 | 75 | 80 | 90 | 100 | 120 |
| 8 | 20 | 30 | 40 | 45 | 55 | 65 | 75 | 85 | 105 |
| 10 | 5 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 90 |
| 15 |  |  |  |  |  | 10 | 20 | 30 | 50 |
| 20 |  |  |  |  |  |  |  |  | 10 |
| 1% soil organic matter |  |  |  |  |  |  |  |  |  |
| 2 | 75 | 85 | 95 | 105 | 115 | 125 | 135 | 150 | 170 |
| 4 | 55 | 70 | 80 | 90 | 100 | 110 | 120 | 130 | 155 |
| 6 | 40 | 50 | 60 | 75 | 85 | 95 | 105 | 115 | 140 |
| 8 | 25 | 35 | 45 | 55 | 70 | 80 | 90 | 100 | 120 |
| 10 | 10 | 20 | 30 | 40 | 50 | 65 | 75 | 85 | 105 |
| 15 |  |  |  |  | 10 | 25 | 35 | 45 | 65 |
| 20 |  |  |  |  |  |  |  | 5 | 25 |
| Round to nearest 5 pounds |  |  |  |  |  |  |  |  |  |

Table R-12.  Potato.  Due to potato disease microorganisms entering a field in manure, manure applications are strongly discouraged where potatoes are or may be in the rotation.

Table R-13.  Nitrogen  recommendations for sorghum.

81

| Soil Nitrate Level |  | Expected Yield |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (bushels per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  | 60 |  | 80 |  | 100 |  | 120 |  | 140 | 160 |  | 180 |  |  | 200 |
| (ppm) |  | Nitrogen to Apply (pounds per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Organic Matter: 3% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 25 |  | 50 |  | 70 |  | 90 |  | 115 |  | 135 |  | 160 | 180 |  |  | 200 |  |
| 4 | 0 |  | 20 |  | 40 |  | 65 |  | 85 |  | 110 |  | 130 | 150 |  |  | 175 |  |
| 6 | 0 |  | 0 |  | 10 |  | 35 |  | 55 |  | 80 |  | 100 | 120 |  |  | 145 |  |
| 8 | 0 |  | 0 |  | 0 |  | 5 |  | 30 |  | 50 |  | 70 | 95 |  |  | 115 |  |
| 10 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 20 |  | 45 | 65 |  |  | 90 |  |
| 12 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 15 | 35 |  |  | 60 |  |
| 14 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 10 |  |  | 30 |  |
| 16 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  | 0 |  |
| 18 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  | 0 |  |
| Organic Matter: 2% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 45 |  | 70 |  | 90 |  | 110 |  | 135 |  | 155 |  | 180 | 200 |  |  | 220 |  |
| 4 | 20 |  | 40 |  | 60 |  | 85 |  | 105 |  | 130 |  | 150 | 170 |  |  | 195 |  |
| 6 | 0 |  | 10 |  | 30 |  | 55 |  | 75 |  | 100 |  | 120 | 140 |  |  | 165 |  |
| 8 | 0 |  | 0 |  | 5 |  | 25 |  | 50 |  | 70 |  | 90 | 115 |  |  | 135 |  |
| 10 | 0 |  | 0 |  | 0 |  | 0 |  | 20 |  | 40 |  | 65 | 85 |  |  | 110 |  |
| 12 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 10 |  | 35 | 55 |  |  | 80 |  |
| 14 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 5 | 30 |  |  | 50 |  |
| 16 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  | 20 |  |
| 18 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  |  | 0 |  |
| Organic Matter: 1% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 65 |  | 90 |  | 110 |  | 130 |  | 155 |  | 175 |  | 200 | 220 |  | 240 |  |  |
| 4 | 40 |  | 60 |  | 80 |  | 100 |  | 125 |  | 145 |  | 170 | 190 |  | 215 |  |  |
| 6 | 10 |  | 30 |  | 50 |  | 75 |  | 95 |  | 120 |  | 140 | 160 |  | 185 |  |  |
| 8 | 0 |  | 1 |  | 25 |  | 45 |  | 70 |  | 90 |  | 110 | 135 |  | 155 |  |  |
| 10 | 0 |  | 0 |  | 0 |  | 20 |  | 40 |  | 60 |  | 85 | 105 |  | 130 |  |  |
| 12 | 0 |  | 0 |  | 0 |  | 0 |  | 10 |  | 30 |  | 55 | 75 |  | 100 |  |  |
| 14 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 3 |  | 25 | 50 |  | 70 |  |  |
| 16 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 20 |  | 40 |  |  |
| 18 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  | 10 |  |  |

References

Table R-14. Nitrogen recommendations for sugar beets in Nebraska

|  |  | Soil Organic Matter (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Residual Soil Nitrate Level |  | 0 to 1.4 | 1.5 to 1.7 |  | 1.8 to 2.1 | > 2.1 |
| (Lbs. per acre, 6 foot sample) | ppm | Nitrogen to Apply (pounds per acre) |  |  |  |  |
| 0 to 5 | 0 to 0.25 | 195 | 185 | 175 |  | 165 |
| 6 to 25 | 0.25 to 1.2 | 175 | 165 | 155 |  | 145 |
| 26 to 45 | 1.2 to 2.1 | 155 | 145 | 135 |  | 125 |
| 46 to 65 | 2.1 to 3.0 | 135 | 125 | 115 |  | 105 |
| 66 to 85 | 3.0 to 3.9 | 115 | 105 | 95 |  | 85 |
| 86 to 105 | 3.9 to 4.8 | 95 | 85 | 75 |  | 65 |
| 106 to 125 | 4.8 to 5.7 | 75 | 65 | 55 |  | 45 |
| 126 to 145 | 5.7 to 6.6 | 55 | 45 | 35 |  | 25 |
| 146 to 165 | 6.6 to 7.5 | 35 | 25 | 0 |  | 0 |
| > 165 | > 7.5 | 0 | 0 | 0 |  | 0 |

Table R-15.  Nitrogen  recommendations for sunflower.

82

| Residual Soil Nitrate Level* | Dryland Expected Yield |  |  |  |  |  | Irrigated Expected Yield |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (pounds per acre) |  |  |  |  |  |  |  |  |  |
|  | 1000 | 1200 | 1400 | 1600 | 1800 |  | 2000 | 2200 | 2400 | 2600 |
| (ppm) | Nitrogen to Apply (pounds per acre) |  |  |  |  |  |  |  |  |  |
| 0 to 1.0 | 30 | 40 | 50 | 60 | 70 |  | 80 | 90 | 100 | 110 |
| 1.0 to 2.0 | 15 | 25 | 35 | 45 | 55 |  | 65 | 75 | 85 | 95 |
| 2.1 to 3.0 |  | 10 | 20 | 30 | 40 |  | 50 | 60 | 70 | 80 |
| 3.1 to 4.0 |  | 0 | 0 | 15 | 25 |  | 35 | 45 | 55 | 65 |
| 4.1 to 5.0 |  | 0 | 0 | 0 | 0 |  | 20 | 30 | 40 | 50 |
| 5.1 to 6.0 |  |  |  |  |  |  | 0 | 15 | 25 | 35 |
| > 6.0 |  |  |  |  |  |  | 0 | 0 | 10 | 20 |
| * Average ppm NO3-N in a 0 to 3-foot sample |  |  |  |  |  |  |  |  |  |  |

Table R-16.  Nitrogen recommendations for spring wheat.

|  | Soil Organic Matter (%) |  |  |
| --- | --- | --- | --- |
| Expected Yield | 1 | 2 | 3 |
| (bushels per acre) | Nitrogen to Apply (pounds per acre) |  |  |
| 30 | 62 | 42 | 22 |
| 50 | 110 | 90 | 70 |
| 70 | 158 | 138 | 118 |

Table R-17.  Recommended nitrogen rates for winter wheat.

| Residual Soil Nitrate Level (Average ppm in 3 ft.) | Wheat Price per Bushel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | $2.50 |  |  |  |  | $3.50 |  |  |
|  | Fertilizer price per pound of nitrogen |  |  |  |  |  |  |  |
|  | $0.15 | $0.20 | $0.25 |  |  | $0.15 | $0.20 | $0.25 |
|  | Nitrogen to apply (pounds per acre) |  |  |  |  |  |  |  |
| 3 | 95 | 81 | 66 |  |  | 108 | 97 | 87 |
| 6 | 64 | 49 | 35 |  |  | 76 | 66 | 55 |
| 9 | 32 | 17 | 3 |  |  | 44 | 34 | 24 |
| 12 | 0 | 0 | 0 |  |  | 13 | 2 | 0 |
| 15 | 0 | 0 | 0 |  |  | 0 | 0 | 0 |

Table R-18.  Recommended rates of phosphorus on alfalfa.

83

| Phosphorus Soil Test Level |  |  | P2O5 to Apply |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Annually | Annually2 | Applied Every 2 years3 |
| Bray-1 | Olsen-P | Relative Level | Irrigated1 | Non-Irrigated |  |
| ppm |  |  | Pounds per acre |  |  |
| 0 to 5 | 0 – 3 | Very low (vl) | 60 | 40 | 80 |
| 6 to 15 | 4 – 10 | Low (l) | 40 | 30 | 60 |
| 16 to 25 | 11 – 17 | Medium (m) | 30 | 20 | 40 |
| > 25 | > 17 | High (h) |  |  |  |
| 1 Established stands where the expected yield is six to eight tons per acre with good water management perform best with annual early spring application. For new seedings of irrigated alfalfa, the farmer should plow down or disk in phosphate fertilizer ahead of seeding at twice the recommended annual rate if soils are low or very low in soil-test phosphorus. This should provide adequate phosphate for the first production year. 2 Except in calcareous (high lime) soils, the producer should plow down or disk in applications ahead of seeding at three times the recommended annual rate for non-irrigated alfalfa. This should meet phosphorus needs for three to four years. 3 On calcareous soils in northeast Nebraska (Crofton and Nora soil series), application ahead of seeding following by top dressing every two years is the most profitable method. |  |  |  |  |  |

Table R-19.  Phosphorus recommendations for barley.

| Phosphorus Soil Test |  |  | P2O5 Application Method |  |
| --- | --- | --- | --- | --- |
| Relative Level | Bray-1 | Olsen-P | Broadcast | Band |
|  | ppm |  | Pounds per acre |  |
| Very low | 0 – 5 | 0 – 3 | 80 | 40 |
| Low | 6 – 15 | 4 – 10 | 60 | 30 |
| Medium | 16 – 25 | 11 – 17 | 40 | 20 |
| High | > 25 | > 17 | 0 | 0 |

Table R-20.  Phosphorus  recommendations for corn.

| Phosphorus Soil Test |  |  | P2O5 to Apply |  |
| --- | --- | --- | --- | --- |
| Bray-1* | Olsen P* | Relative | Broadcast | Band** |
| ppm |  | Level | Pounds per acre |  |
| References 0 to 5 | 0 to 3 | Very low (vl) | 80 | 40 |
| 6 to 15 | 4 to 10 | Low (l) | 40 | 20 |
| 16 to 24 | 11 to 16 | Medium (m) | 0 |  |
| 25 to 30 | 17 to 20 | High (h) | 0 |  |
| > 30 | > 20 | Very high (vh) | 0 | 0 |
| *Phosphorus tests: Bray-1 for acid and neutral soils; Olsen P for calcareous soils (pH 7.3 or greater). ** Applied in a band preplant or beside the row at planting.  Applying 10 to 20 pounds per acre P2O5 with 5 to 10 pounds per acre nitrogen in a band at planting may increase early growth on these soils. See NebGuide G77-361, “Using Starter Fertilizers for Corn, Grain Sorghum and Soybeans.” |  |  |  |  |

Table R-21.  Phosphorus  recommendations for dry edible beans.

84

| Phosphorus Soil Test |  |  |  | P2O5 Application Rate |  |
| --- | --- | --- | --- | --- | --- |
| Relative Level | Bray-1 | Olsen-P |  | Broadcast | Band |
|  | ppm |  |  | Pounds per acre |  |
| Low | 0 – 5 | 0 – 3 |  | 20 | 40 |
| Medium | 6-15 | 4 – 7 |  | 10 | 20 |
| High | >15 | >7 |  | 0 | 0 |

Table R-22.  Phosphorus recommendations for dryland and irrigated grasslands.

| Phosphorus Soil Test |  |  | Dryland P2O5 to Apply | Irrigated P2O5 to Apply |  |
| --- | --- | --- | --- | --- | --- |
| Relative Level | Bray-1 | Olsen-P |  | Grass | Grass-legume |
|  | ppm |  |  | Pounds per acre |  |
| Very Low | 0 – 5 | 0 – 3 | 40 | 60 | 90 |
| Low | 6 – 15 | 4 – 10 | 20 | 40 | 60 |
| Medium | 16 – 25 | 10 – 17 | 10 | 20 | 30 |
| High | >25 | >17 | 0 | 0 | 0 |

Table R-23.  Phosphorus recommendations for millet.

| Phosphorus Soil Test |  |  | P2O5 to Apply |  |
| --- | --- | --- | --- | --- |
| Bray-1 P | Olsen-P | Relative Level | Broadcast | Band |
| ppm |  |  | Pounds per acre |  |
| <10 | < 5 | Very low | 80 | 40 |
| 10 to 15 | 5 to 8 | Low | 40 | 20 |
| 16 to 20 | 8 to 12 | Medium | 20 | 10 |
| > 20 | > 12 | High | 0 | 0 |

Table R-24.  Phosphorus recommendations for oats and other spring small grains.

| Phosphorus Soil Test |  |  | P2O5 Application Method |  |
| --- | --- | --- | --- | --- |
| Relative Level | Bray-1 P | Olsen-P | Broadcast | Band |
|  | ppm |  | Pounds per acre |  |
| Very low | 0 – 5 | 0 – 3 | 80 | 40 |
| Low | 6 – 15 | 4 – 10 | 60 | 30 |
| Medium | 16 – 25 | 11 – 17 | 40 | 20 |
| High | >25 | >17 | 0 | 0 |

Table R-25.  Phosphorus recommendations for popcorn.

85

| Phosphorus Soil Test |  |  | P2O5 to Apply |  |
| --- | --- | --- | --- | --- |
| Bray-1 P* | Olsen-P* | Relative Level | Broadcast | Band ** |
| ppm |  |  | Pounds per acre |  |
| 0 to 5 | 0 to 3 | Very low (vl) | 80 | 40 |
| 6 to 15 | 4 to 10 | Low (l) | 40 | 20 |
| 16 to 24 | 11 to 16 | Medium (m) | 0 |  |
| 25 to 30 | 17 to 20 | High (h) | 0 |  |
| >30 | >20 | Very high (vh) | 0 | 0 |
| * Phosphorus tests: Bray-1 for acid and neutral soils; Olsen-P for calcareous soil (pH 7.2 or greater). ** Applied in a band preplant or beside the row at planting.  Applying 10 to 20 pounds per acre P2O5 with 5 to 10 pounds per acre nitrogen in a band at planting may increase early growth on these soils. See NebGuide G77-631, “Using Starter Fertilizers for Corn, Grain Sorghum and Soybeans.” |  |  |  |  |

Table R-26 Potato.  Due to potato disease microorganisms entering a field in manure, manure applications are strongly discouraged where potatoes are or may be in the rotation.

Table R-27.  Phosphorus recommendations for sorghum.

| Phosphorus Soil Test | P2O5 to Apply |  |
| --- | --- | --- |
| Bray-1 P | Broadcast | Band |
| ppm | Pounds per acre |  |
| 0 to 5 | 80 | 40 |
| 6 to 15 | 40 | 20 |
| 16 to 25 | 0 | 0 |
| > 25 | 0 | 0 |

Table R-28.  Phosphorus recommendations for soybean.

| Phosphorus Soil Test |  | P2O5 |
| --- | --- | --- |
| Bray-1 P | Olsen-P | to Apply |
| ppm |  | Pounds per acre |
| 0 to 4 | 0 to 3 | 65 |
| References 6 to 8 | 4 to 5 | 40 |
| 9 to 12 | 6 to 8 | 20 |
| > 12 | > 8 | 0 |

Table R-29.  Phosphorus  recommendations for sugar beets.

| Phosphorus Soil Test |  |  | P2O5 |
| --- | --- | --- | --- |
| Bray-1 P | Olsen-P* | Relative Level | to Apply |
| ppm |  |  | Pounds per acre |
| 0 to 5 | 0 to 3 | Very low | 100 |
| 6 to 10 | 4 to 7 | Low | 75 |
| 11 to 15 | 7 to 10 | Marginal | 50 |
| >15 | >10 | Adequate | 0 |
| * For Mitchell soils with an Olsen P level between 11 and 16 ppm, apply 25 pounds P2O5 per acre. |  |  |  |

Table R-30.  Phosphorus recommendations for sunflower.

86

| Phosphorus Soil Test |  | P2O5 to Apply |  |
| --- | --- | --- | --- |
| Bray-1 P* | Olsen-P* | Broadcast | Band |
| ppm |  | Pounds per acre |  |
| 0 to 5 | 0 to 4 | 60 | 30 |
| 6 to 15 | 5 to 10 | 40 | 20 |
| 16 to 25 | 11 to 15 | 20 | 10 |
| >25 | >15 | 0 | 0 |
| * Phosphorus tests: Bray-1 for acid and neutral soils; Olsen-P for calcareous soils. |  |  |  |

Table R-31.  Phosphorus recommendations for spring wheat.

| Phosphorus Soil Test |  |  | P2O5 Application Method |  |
| --- | --- | --- | --- | --- |
| Relative Level | Bray-1 P | Olsen-P | Broadcast | Band |
|  | ppm |  | Pounds per acre |  |
| Very low | 0 – 5 | 0 – 3 | 80 | 40 |
| Low | 6 – 15 | 4 – 10 | 60 | 30 |
| Medium | 16 – 25 | 11 – 17 | 40 | 20 |
| High | >25 | >17 | 0 | 0 |

Table R-32.  Most profitable phosphorus application rates for winter wheat in Nebraska for different application methods and expected yield.

| Phosphorus Soil Test |  | Yield Level (bushels/acre) |  |  |
| --- | --- | --- | --- | --- |
| Bray-1 P | Olsen-P | 40 | 50 | 70 |
| ppm |  | P2O5 to apply pounds/ac* |  |  |
|  |  | Broadcast |  |  |
| 5 | 3 | 50 | 60 | 70 |
| 10 | 7 | 20 | 25 | 40 |
| 15 | 10 | 0 | 10 | 25 |
| 20 | 13 | 0 | 0 | 10 |
| 25 | 17 | 0 | 0 | 0 |
| 30 | 20 | 0 | 0 | 0 |
|  |  | Row or dual application |  |  |
| 5 | 3 | 35 | 50 | 70 |
| 10 | 7 | 10 | 25 | 45 |
| 15 | 10 | 0 | 10 | 30 |
| 20 | 13 | 0 | 0 | 20 |
| 25 | 17 | 0 | 0 | 15 |
| 30 | 20 | 0 | 0 | 10 |
| * Based on $4 per bushel wheat and 30 cents per pound of P2O5 and a soil pH of 7.0 (broadcast only). |  |  |  |  |

# MANURE APPLICATOR CALIBRATION GUIDE

1

1. Spreader Capacity is Known.  From chart below, select 1) Spreader Capacity: __________ lbs. or gallons; 2) Distance traveled (length) to empty spreader: _________ feet; and 3) Spread pattern width or distance between individual passes:  _________ feet.  4)  Intersection indicates application rate:________.  If appropriate values cannot be found in table below:  Rate per acre = Spreader Capacity x 43560 / (Width X Length).  Example:  3000 gallon tank spreader that makes a pass every 4 30”corn rows (10 feet) and empties spreader in 1200 feet is applying 11,000 gallons per acre.

3

| Spead Width | 2000 Gallon tank |  |  |  |  |  | 2500 gallon tank |  |  |  |  |  | 3000 gallon tank |  |  |  |  |  | 3500 gallon tank |  |  |  |  |  | 4000 gallon tank |  |  |  |  |  | 4500 gallon tank |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ |
| Length | Liquid manure application rate (1000’s of gallons per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 600’ | 15 | 10 | 7 | 6 | 5 | 4 | 18 | 12 | 9 | 7 | 6 | 5 | 22 | 15 | 11 | 7 | 5 | 4 | 25 | 17 | 13 | 8 | 6 | 5 | 29 | 19 | 15 | 10 | 7 | 6 | 33 | 22 | 16 | 11 | 8 | 7 |
| 800’ | 11 | 7 | 5 | 4 | 4 | 3 | 14 | 9 | 7 | 5 | 5 | 4 | 16 | 11 | 8 | 5 | 4 | 3 | 19 | 13 | 10 | 6 | 5 | 4 | 22 | 15 | 11 | 7 | 5 | 4 | 25 | 16 | 12 | 8 | 6 | 5 |
| 2 1000’ | 9 | 6 | 4 | 3 | 3 | 2 | 11 | 7 | 5 | 4 | 4 | 3 | 13 | 9 | 7 | 4 | 3 | 3 | 15 | 10 | 8 | 5 | 4 | 3 | 17 | 12 | 9 | 6 | 4 | 3 | 20 | 13 | 10 | 7 | 5 | 4 |
| 1200’ | 7 | 5 | 4 | 3 | 2 | 2 | 9 | 6 | 5 | 4 | 3 | 3 | 11 | 7 | 5 | 4 | 3 | 2 | 13 | 8 | 6 | 4 | 3 | 3 | 15 | 10 | 7 | 5 | 4 | 3 | 16 | 11 | 8 | 5 | 4 | 3 |
| 1400’ | 6 | 4 | 3 | 2 | 2 | 2 | 8 | 5 | 4 | 3 | 3 | 2 | 9 | 6 | 5 | 3 | 2 | 2 | 11 | 7 | 5 | 4 | 3 | 2 | 12 | 8 | 6 | 4 | 3 | 2 | 14 | 9 | 7 | 5 | 4 | 3 |
| 1600’ | 5 | 4 | 3 | 2 | 2 | 2 | 7 | 5 | 3 | 3 | 2 | 2 | 8 | 5 | 4 | 3 | 2 | 2 | 10 | 6 | 5 | 3 | 2 | 2 | 11 | 7 | 5 | 4 | 3 | 2 | 12 | 8 | 6 | 4 | 3 | 2 |
| 1800’ | 5 | 3 | 2 | 2 | 2 | 1 | 6 | 4 | 3 | 2 | 2 | 2 | 7 | 5 | 4 | 2 | 2 | 1 | 8 | 6 | 4 | 3 | 2 | 2 | 10 | 6 | 5 | 3 | 2 | 2 | 11 | 7 | 5 | 4 | 3 | 2 |
| 2000’ | 4 | 3 | 2 | 2 | 1 | 1 | 5 | 4 | 3 | 2 | 2 | 2 | 7 | 4 | 3 | 2 | 2 | 1 | 8 | 5 | 4 | 3 | 2 | 2 | 9 | 6 | 4 | 3 | 2 | 2 | 10 | 7 | 5 | 3 | 2 | 2 |
| 2500’ | 3 | 2 | 2 | 1 | 1 | 1 | 4 | 3 | 2 | 2 | 1 | 1 | 5 | 3 | 3 | 2 | 1 | 1 | 6 | 4 | 3 | 2 | 2 | 1 | 7 | 5 | 3 | 2 | 2 | 1 | 8 | 5 | 4 | 3 | 2 | 2 |
| 3000’ | 3 | 2 | 1 | 1 | 1 | 1 | 4 | 2 | 2 | 1 | 1 | 1 | 4 | 3 | 2 | 1 | 1 | 1 | 5 | 3 | 3 | 2 | 1 | 1 | 6 | 4 | 3 | 2 | 1 | 1 | 7 | 4 | 3 | 2 | 2 | 1 |

| Spead Width | 5000 Gallon tank |  |  |  |  |  | 5500 gallon tank |  |  |  |  |  | 6000 gallon tank |  |  |  |  |  | 3 ton spreader (90 bushel) |  |  |  |  |  | 4 ton spreader (120 bushel) |  |  |  |  |  | 6 ton spreader (175 bushel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 30’ | 40’ | 50’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ |
| Length | Liquid manure application rate (1000’s of gallons per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Solid manure application rate (tons per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 600’ | 36 | 24 | 18 | 12 | 9 | 7 | 40 | 27 | 20 | 13 | 10 | 8 | 44 | 29 | 22 | 15 | 11 | 9 | 22 | 15 | 11 | 9 | 7 | 6 | 29 | 19 | 15 | 12 | 10 | 8 | 44 | 29 | 22 | 17 | 15 | 12 |
| 800’ | 27 | 18 | 14 | 9 | 7 | 5 | 30 | 20 | 15 | 10 | 7 | 6 | 33 | 22 | 16 | 11 | 8 | 7 | 16 | 11 | 8 | 7 | 5 | 5 | 22 | 15 | 11 | 9 | 7 | 6 | 33 | 22 | 16 | 13 | 11 | 9 |
| 1000’ | 22 | 15 | 11 | 7 | 5 | 4 | 24 | 16 | 12 | 8 | 6 | 5 | 26 | 17 | 13 | 9 | 7 | 5 | 13 | 9 | 7 | 5 | 4 | 4 | 17 | 12 | 9 | 7 | 6 | 5 | 26 | 17 | 13 | 10 | 9 | 7 |
| 1200’ | 18 | 12 | 9 | 6 | 5 | 4 | 20 | 13 | 10 | 7 | 5 | 4 | 22 | 15 | 11 | 7 | 5 | 4 | 11 | 7 | 5 | 4 | 4 | 3 | 15 | 10 | 7 | 6 | 5 | 4 | 22 | 15 | 11 | 9 | 7 | 6 |
| 1400’ | 16 | 10 | 8 | 5 | 4 | 3 | 17 | 11 | 9 | 6 | 4 | 3 | 19 | 12 | 9 | 6 | 5 | 4 | 9 | 6 | 5 | 4 | 3 | 3 | 12 | 8 | 6 | 5 | 4 | 4 | 19 | 12 | 9 | 7 | 6 | 5 |
| 1600’ | 14 | 9 | 7 | 5 | 3 | 3 | 15 | 10 | 7 | 5 | 4 | 3 | 16 | 11 | 8 | 5 | 4 | 3 | 8 | 5 | 4 | 3 | 3 | 2 | 11 | 7 | 5 | 4 | 4 | 3 | 16 | 11 | 8 | 7 | 5 | 5 |
| 1800’ | 12 | 8 | 6 | 4 | 3 | 2 | 13 | 9 | 7 | 4 | 3 | 3 | 15 | 10 | 7 | 5 | 4 | 3 | 7 | 5 | 4 | 3 | 2 | 2 | 10 | 6 | 5 | 4 | 3 | 3 | 15 | 10 | 7 | 6 | 5 | 4 |
| 2000’ | 11 | 7 | 5 | 4 | 3 | 2 | 12 | 8 | 6 | 4 | 3 | 2 | 13 | 9 | 7 | 4 | 3 | 3 | 7 | 4 | 3 | 3 | 2 | 2 | 9 | 6 | 4 | 3 | 3 | 2 | 13 | 9 | 7 | 5 | 4 | 4 |
| 2500’ | 9 | 6 | 4 | 3 | 2 | 2 | 10 | 6 | 5 | 3 | 2 | 2 | 10 | 7 | 5 | 3 | 3 | 2 | 5 | 3 | 3 | 2 | 2 | 1 | 7 | 5 | 3 | 3 | 2 | 2 | 10 | 7 | 5 | 4 | 3 | 3 |
| 3000’ | 7 | 5 | 4 | 2 | 2 | 1 | 8 | 5 | 4 | 3 | 2 | 2 | 9 | 6 | 4 | 3 | 2 | 2 | 4 | 3 | 2 | 2 | 1 | 1 | 6 | 4 | 3 | 2 | 2 | 2 | 9 | 6 | 4 | 3 | 3 | 2 |

| Spread Width | 8 ton spreader (230 bushel) |  |  |  |  |  | 10 ton spreader (290 bushel) |  |  |  |  |  | 12 ton spreader (350 bushel) |  |  |  |  |  | 14 ton spreader (410 bushel) |  |  |  |  |  | 16 ton spreader (470 bushel) |  |  |  |  |  | 18 ton spreader (530 bushel) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ | 10’ | 15’ | 20’ | 25’ | 30’ | 35’ |
| Length | Solid manure application rate (tons per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Solid manure application rate (tons per acre) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 600’ | 58 | 39 | 29 | 23 | 19 | 17 | 73 | 48 | 36 | 29 | 24 | 21 | 87 | 58 | 44 | 35 | 29 | 25 | 102 | 68 | 51 | 41 | 34 | 29 | 116 | 77 | 58 | 46 | 39 | 33 | 131 | 87 | 65 | 52 | 44 | 37 |
| 800’ | 44 | 29 | 22 | 17 | 15 | 12 | 54 | 36 | 27 | 22 | 18 | 16 | 65 | 44 | 33 | 26 | 22 | 19 | 76 | 51 | 38 | 30 | 25 | 22 | 87 | 58 | 44 | 35 | 29 | 25 | 98 | 65 | 49 | 39 | 33 | 28 |
| 1000’ | 35 | 23 | 17 | 14 | 12 | 10 | 44 | 29 | 22 | 17 | 15 | 12 | 52 | 35 | 26 | 21 | 17 | 15 | 61 | 41 | 30 | 24 | 20 | 17 | 70 | 46 | 35 | 28 | 23 | 20 | 78 | 52 | 39 | 31 | 26 | 22 |
| 1200’ | 29 | 19 | 15 | 12 | 10 | 8 | 36 | 24 | 18 | 15 | 12 | 10 | 44 | 29 | 22 | 17 | 15 | 12 | 51 | 34 | 25 | 20 | 17 | 15 | 58 | 39 | 29 | 23 | 19 | 17 | 65 | 44 | 33 | 26 | 22 | 19 |
| 1400’ | 25 | 17 | 12 | 10 | 8 | 7 | 31 | 21 | 16 | 12 | 10 | 9 | 37 | 25 | 19 | 15 | 12 | 11 | 44 | 29 | 22 | 17 | 15 | 12 | 50 | 33 | 25 | 20 | 17 | 14 | 56 | 37 | 28 | 22 | 19 | 16 |
| 1600’ | 22 | 15 | 11 | 9 | 7 | 6 | 27 | 18 | 14 | 11 | 9 | 8 | 33 | 22 | 16 | 13 | 11 | 9 | 38 | 25 | 19 | 15 | 13 | 11 | 44 | 29 | 22 | 17 | 15 | 12 | 49 | 33 | 25 | 20 | 16 | 14 |
| 1800’ | 19 | 13 | 10 | 8 | 6 | 6 | 24 | 16 | 12 | 10 | 8 | 7 | 29 | 19 | 15 | 12 | 10 | 8 | 34 | 23 | 17 | 14 | 11 | 10 | 39 | 26 | 19 | 15 | 13 | 11 | 44 | 29 | 22 | 17 | 15 | 12 |
| 2000’ | 17 | 12 | 9 | 7 | 6 | 5 | 22 | 15 | 11 | 9 | 7 | 6 | 26 | 17 | 13 | 10 | 9 | 7 | 30 | 20 | 15 | 12 | 10 | 9 | 35 | 23 | 17 | 14 | 12 | 10 | 39 | 26 | 20 | 16 | 13 | 11 |
| 2500’ | 14 | 9 | 7 | 6 | 5 | 4 | 17 | 12 | 9 | 7 | 6 | 5 | 21 | 14 | 10 | 8 | 7 | 6 | 24 | 16 | 12 | 10 | 8 | 7 | 28 | 19 | 14 | 11 | 9 | 8 | 31 | 21 | 16 | 13 | 10 | 9 |
| 3000’ | 12 | 8 | 6 | 5 | 4 | 3 | 15 | 10 | 7 | 6 | 5 | 4 | 17 | 12 | 9 | 7 | 6 | 5 | 20 | 14 | 10 | 8 | 7 | 6 | 23 | 15 | 12 | 9 | 8 | 7 | 26 | 17 | 13 | 10 | 9 | 7 |

87

2. Spreader Capacity is Unknown.

88

- Cut three or more sheets of equally sized plastic.  22 square feet (3’ x 7’4” or 4’ X 5’6”) is preferred size.

- Weigh empty 5 gallon bucket plus one plastic sheet on a scale:  ________________ lbs.

- Lay sheets in field with edges secured by stones or other heavy objects.

- Drive tractor at normal speeds and discharge manure at typical rates over plastic sheets.  Record tractor gear:  ___________,  engine RPM:  _____________ , and spreader settings:  _____________

Plastic Sheets

- Check the sheet.  Did a reasonably representative application rate fall on the plastic sheet?

- Carefully fold individual sheets without losing manure and place each sheet in separate buckets.

Weigh each bucket.    Bucket 1:  ________lbs.    Bucket 2:  ________lbs.    Bucket 3:  ________lbs.

- Subtract weight of empty bucket and plastic (step b) to determine net manure weight is each bucket.

Net manure weight for  Bucket 1:  ________lbs.    Bucket 2:  ________lbs.    Bucket 3:  ________lbs.

- Calculate average weight of buckets.	Average Net Manure Weight: ________lbs.

- Calculate application rate. Tons per Acre  =  (Net Manure Weight  X  22)  ÷  area of plastic sheet (ft2)

If plastic sheet = 22 ft2, then  Tons per Acre  =  Net Manure Weight

3. Pivot Calibration

A.  If Pivot Flow Rate Is Known:

- Estimate pumping time: _______________ hours

- Estimate water flow rate: ______________ gallons per minute

- Estimate acres covered:  ______________ acres

- Estimate application rate:

Pumping Time  X  Flow Rate		X

Inches (or ac–in/ac)  =                                                    =			                                    =  ________ in.

Acres X  450		  X	450

B. If Pivot Flow Rate Is NOT Known:

- Place 4 to 6 rain gauges (pans or straight sided plastic cups will also work) in line with the pivot center point at roughly equally spaced intervals.  Placement on access road away from crop canopy is preferred.

- Measure depth in rain gauges and calculate average.

Gauge #1: _____ in.    #2: _____ in.   #3: _____ in.   #4: _____ in.   #5: _____ in.    #6.______ in.

Average Depth:  _______inches

OR

- Identify Rated Pump Pressure and Flow Rate: ________ psi at _________ gpm

- Identify Actual Pump Pressure:________ psi

- Estimate Actual Flow Rate.

*

GPM actual =  GPM rated X       P actual  /  P rated  =  ______  X     ______  /  ______  = ______ gpm

- Substitute actual Flow Rate from c. into the Flow Rate space in d. of  “A. If Flow Rate Is Known” and complete calculation of application rate.

*  Square Root

89

ENVIRONMENTAL GUIDANCE DOCUMENT

00-046	April, 2000

# Comprehensive Nutrient Management Plan

Comprehensive nutrient management plan (CNMP) requirements can be found in Title 130, Chapters 3, 10, and 11.  There may be additional requirements based on the actual construction, operation, and maintenance of a particular livestock waste control facility.  Sludge management, odor, equipment capacity limitations, and closure concerns should also be considered.

## Land Application Area

- Legal description of the planned waste application areas.

- Number of useable acres.

- Slope and soil type.

- Cropping practices and historic yields.

- Distance to surface water.

- Location of wetlands.

- Use by other operations.

- Easements as necessary with landowner(s)’ name, address, site legal description, number of acres, and use agreement signed by the landowner(s).  Easements or access agreements are also required when access to an application site requires waste to be conveyed across property not owned by the operation, including county right-of-ways.

## Sampling, Analysis, and Application Plan

- Waste sampling methods, frequency of sampling, and analysis procedures.

References

Soil sampling methods, frequency of sampling, and analysis procedures with a description of sample areas (generally no more than 40 acres per sample), sample depth (generally 6 to 8 inch composite for phosphorus and 24 inches or greater for nitrogen).  Include test methods (such as Bray 1, Olson, Mehlich, etc. for phosphorus).

- Planned application rates, application methods, and frequencies.  Plan must include the method used to determine the agronomic rate for nitrogen.

- If an irrigation system, the plan must include the type and location of any mechanical devices (pipe, pumps, valves, etc.).  Plan must indicate if system will be disconnected from the irrigation water source during livestock waste application.

## Record Keeping

- Locations where livestock waste applied.

- Estimates and analyses of nutrient value of livestock waste used to determine application rates.

- Quantities of livestock wastes applied to application areas.

- Record of sample results.

- Record of any livestock waste sold or given away.

Location of wetlands (determined by NRCS) on the operation or application areas.

90

- Maintain records for 5 years (or longer if required in approved CNMP).  Longer periods may be required in some instances due to the type of livestock waste control facility or other relevant concerns.

## Best Management Practices (BMP)

- Facilities dewatered to maintain required minimum storage.

- Land application or stockpiling in a manner that will not contribute to water pollution.

- Sample application area for available phosphorus prior to use and as scheduled.

- A plan describing BMP to minimize odor is required of all Class II, III, and IV facilities.

## Reporting and Setback Requirements

- Report to the NDEQ all land application area soil analyses in which available phosphorus exceeds 150 parts per million.  An alternative level for reporting may be required by NDEQ based on sampling and analysis methods.

- Restrict livestock waste application within 30 feet of any streams, lakes, and impounded waters.

- NDEQ may require additional restrictions for waste application within 100 feet of stream, lakes, or impounded waters.

## Other Considerations

## Monitoring of irrigation distribution system and other waste handling equipment.

- Application methods suitable to the operation including the storage capacity of the facility, weather, type of waste, soils, cropping practices, and facility management.

- Dead animals or carcasses shall not be placed in livestock waste control facilities or land applied with livestock waste.

NDEQ regulations for livestock waste control facility permits are contained in Title 130.  A full copy of title 130 is available at http://www.deq.state.ne.us/, under “Rules and Regulations” option.

NDEQ fact sheets and guidance documents for livestock waste control facility permits can be found at http://www.deq.state.ne.us/, under “Publications and Forms” option and “Livestock Program” option.

Questions?  Contact:  Nebraska Department of Environmental Quality, Agriculture Section, P.O. Box 98922, Lincoln, NE  68509-8922; phone (402) 471-4239. Visit our web site at http://www. deq.state.ne.us.

Issued in furtherance of Cooperative Extension work, Acts of May 8 and June 30, 1914, in cooperation with the

U.S. Department of Agriculture. Elbert C. Dickey, Dean and Director of Cooperative Extension,

University of Nebraska, Institute of Agriculture and Natural Resources.

University of Nebraska Cooperative Extension educational programs abide with the non-discrimination policies of the

University of Nebraska-Lincoln and the United States Department of Agriculture

.

All calculations of land requirements, manure storage capacity and related estimates are based upon a facility’s maximum one-time number of livestock for which the facility will be permitted.

All calculations of land requirements, manure storage capacity and related estimates are based upon a facility’s maximum one-time number of livestock  and maximum days per year the facility could be occupied (usually 365 days) for which the facility will be permitted by NDEQ.

Not required for NDEQ permit application.

Olsen soil test should only be used for soils with pH above 7.4.

2 This Web site provides an alternative method for completing the calculations

in Forms 6 through 12.

This estimate of nitrogen availability includes assumptions for fraction of the manure nitrogen that is in organic and ammonium forms.  This availability factor is acceptable for estimating land requirements for a permit application.  However, it should not be used for estimating actual manure application rates.  See Figure 1 (page 48) for availability factors for calculating application rates.

Soil test reports should be kept on file for five years with all other records related to manure storage and application.

The soils and manure analysis reports should be maintained on file for five years in a location with all other land application and manure storage records.